# Formal Modeling and Analysis of an Audio/Video Protocol:

An Industrial Case Study Using UPPAAL

Klaus Havelund

Arne Skou

Kim Guldstrand Larsen

BRICS,Aalborg              University,Denmark

{havelund,ask,kgl}@cs.auc.dk

Kristian Lund

Bang   &Olufsen,Denmark klu@bang-olufsen.dk

Abstract

A formal and automatic verification of a real-life proto- col is presented.The protocol,about 2800  lines of assem- bler code,has been used in products from the audio/video company Bang &Olufsen throughout more than a decade, and its purpose is to control the transmission of messages between audio/video components over a single bus.Such communications may collide,and one essential purpose of the protocol is  to  detect such  collisions.The functioning is  highly  dependent  on  real-time  considerations.Though the protocol was known to be faulty in that messages were lost occasionally,the protocol was too complicated in order for Bang &Olufsen to locate the bug using normal testing. However;using the real-time verification tool UPPAAL,an error trace was automatically generated,which caused the detection  of"the  error"in  the  implementation.The  error was corrected and the correction was automatically proven correct,again  using  UPPAAL.A future,and  more  auto- mated,version of the protocol,where this error is fatal,will incorporate the correction.Hence,this work is an elegant demonstration of how model checking has had an impact on practical software development.The effort of modeling this protocol has in addition generated a number ofsugges- tions for enriching the UPPAAL language.Hence,it's also an excellent example of the reverse impact.

##### 1.Introduction

Since the basic results by Alur,Courcoubetis and Dill [1,2]on decidability of model checking for real-time  sys- tems  with  dense  time,a  number  of  tools  for  automatic verification of hybrid and real-time systems have emerged [5,10,8].These tools have by now reached  a  state,where they are mature enough for application on industrial case-

studies as we hope to demonstrate in this paper.

One such tool is the real-time verification tool UPPAAL¹  [5]developed  jointly   by   BRICS²at  Aalborg  University and Department of Computing Systems at Uppsala Univer- sity.The tool provides  support  for  automatic  verification of safety and bounded liveness properties of real-time sys- tems,and it contains a number of additional features includ- ing graphical interfaces for designing and simulating system models.The tool has been applied successfully to a num- ber  of  case-studies   [13,3,4,12,7]which  can  roughly  be divided in two classes:real-time controllers and real-time communication protocols.

Industrial developers of embedded systems have been  following the above work with great interest,because the real-time aspects of concurrent systems can be extremely difficult to analyse during the design and implementation phase.One   such   company   is   Bang   &Olufsen   -having development and production of fully integrated home au- dio/video systems as a main activity.

In  1996,BRICS  and  Bang  &Olufsen  (B&O)agreed  to collaborate on a case study based on one of the company's existing protocols for audio/video device control.The pro- tocol was of interest for three reasons:Firstly,it contained an unexplained error which occasionally caused data loss. The source of this error was unknown to everyone,includ- ing   B&O,prior   to   the   exercise.That   is,normal   testing had  not  been  sufficient  to  identify  the  wrong  code.Our goal  should  be  to  explain  the  error.Secondly,the  proto- col documentation was very low level(consisting solely of assembler listings and flow charts)-so the company could  expect an improved documentation as a byproduct of the work.Thirdly,B&O is about to move(a corrected version

1See URL:http://www.docs.u.se/docs/rtmvuppal/index.shtml for information  about  UPPAAL.

2BRICS-Basic  Research  in  Computer  Science  -is  a  basic  research centre funded by the danish government at Aarhus and Aalborg University.

2

0-8186-8268-X/97$10.00     ◎1997     IEEE

Authorized licensed use limited to:BEIHANG UNIVERSITY.Downloaded on March 21,2026 at 09:02:51 UTC from IEEE Xplore.Restrictions apply.

of)the protocol to a different platform;thus the case-study will test the benefits of the modeling and verification abili- ties of UPPAAL in a realistic development process.Finally, the company had no problems in publishing the results in full detail afterwards.Although the protocol is designed for use in audio/video networks,it is a general purpose protocol applicable also in other contexts.

This paper reports the preliminary results of our collab- oration.We describe how the UPPAAL tool has been ap- plied in constructing a model of the current protocol imple- mentation.The model was developed via 5 major iteration steps during 3 months,where each new step was motivated by further clarification of the implementation-obtained by simulation,trial  verification,discussions  and  code  inspec- tion.In the final model,accepted by B&O as valid with re- spect to the current implementation,we identified a timing error in the collisior detection of the protocol implementa- tion(via diagnostic information provided automatically by UPPAAL).Finally,a corrected version of the protocol was suggested  and  afterwards  successfully  verified.For  each model version,the verification was performed on a suitably reduced model,in order to be manageable by the tool while still allowing the error to be identified.

During the development of models,we found that the notion of timed autcmata and their graphical representation  served extremely well as communication means between the industrial protocol designer and the tool expert doing the  simulation  and  verification.In  addition,the  graphical simulation features of UPPAAL lead to fast detection of sev- eral(obvious)errors in the early models.

The resulting protocol documentation consists of 9 timed automata(a few pages of drawings).This is shorter by an order of magnitude than the original documentation,i.e.a few pages of timed automatons versus 2800 lines of assem- bler code and 3 pages of flow charts.Most of the origi-  nal information was immediately available-either via the flowcharts or through discussions.However,a few times we had to walk through the assembler code in order to obtain precise information,The lack of a model(formal or infor- mal)and the fact that the diagnostic trace³of the protocol consisted of close to 2000 transitions-steps,indicates that the error probably would not have been found without the tool assistance.In fact,by using the diagnostic information from the tool,it was possible to provoke the error in B&O's laboratory.The paper is organized as follows:In sections 2 and 3,we present the UPPAAL tool and the B&O protocol. In section 4 we pre:sent our model of the existing protocol, and in sections 5 arid 6 we present the identification of the protocol error and its correction.Section 7 provides con- cluding remarks,evaluates the UPPAALtool in retrospective and points out further work.

3guaranteed by UPPAAL to be the shortest such.

2.The    UPPAAL  model    and    tool

UPPAAL is a tool box for symbolic simulation and au-  tomatic verification of real-timed systems modeled as net- works  of timed  automata  [2]extended  with  integer  vari- ables.More  precisely,a  model  consists  of  a   collection of non-deterministic processes with finite control structure and real-valued  clocks  communicating  through  channels and  shared  integer  variables.The  tool  box  is  developed in collaboration between BRICS at Aalborg University and Department of Computing Systems at Uppsala University, and has been applied to several case-studies[13,3,4,12,7].

The current version of UPPAAL is implemented in C++, XFORMs and MOTIF and includes the following main fea- tures:

·A graphical interface based on Autograph[6]allowing graphical descriptions of systems.

● A compiler transforming graphical descriptions into a textual programming format.

●A simulator,which provides a graphical visualization and recording of the possible dynamic behaviors of a system description.This allows for inexpensive fault detection in the early modeling stages.

● A model checker for automatic verification of safety and bounded-liveness properties by on-the-fly reach- ability analysis.

·Generation of (shortest)diagnostic traces in case veri- fication of a particular real-time system fails.The di- agnostic traces may be graphically visualized using the simulator.

A system description(or model)in UPPAAL consists of a  collection of automata modeling the finite control structures of the  system.In  addition the model uses  a finite  set  of (global)real-valued clocks and integer variables.

Consider  the  model  of  figure   1.     The  model  con- sists  of  two  components  A  and  B  with  control  nodes {A0,A1,A2,A3}and     {B0,B1,B2,B3}respectively.In    ad- dition to these discrete control structures,the model uses two clocks x and y,one integer variable n and a channel a for communication.

The edges ofthe automata are decorated with three types  of labels:a guard,expressing a condition on the values of clocks and integer variables that must be satisfied in order for the edge to be taken;a synchronization action which is performed when the edge is taken forcing as in CCS [15] synchronization with another component on a complemen- tary action⁴,and finally a number of clock resets and as- signments to integer variables.All three types of labels are

4Given   a    channel   name    a,a!and   a?denote    complementary    actions corresponding  to  sending  respectively  receiving  on  the  channel  a.

3

Authorized licensed use limited to:BEIHANG UNIVERSITY.Downloaded on March 21,2026 at 09:02:51 UTC from IEEE Xplore.Restrictions apply.

Figure 1.An example UPPAAL model

optional:absence of a guard is interpreted as the condition true,and absence of a synchronization action indicates an internal  (non-synchronizing)edge  similar  to  T-transitions in  CCS.Reconsider  figure   1.Here  the  edge  between  A0 and A1 can only be taken,when the value of the clock y is greater than or equal to 3.When the edge is taken the action a!is performed thus insisting on  synchronization with B on the complementary action a?;that is for A to take the edge in question,B must simultaneously be able to take the edge  from  B0  to   B1.Finally,when  taking  the   edge,the clocky is reset to 0.The edge between A2 and A3 can only be taken if n equals 5.

In  addition,control  nodes  may  be  decorated  with  so- called  invariants,which  express  constraints  on  the  clock values in order for control to remain in a particular node. Thus,in figure 1,control can only remain in A0 as long as the value of y is no more than 6.

Formally,states of a UPPAAL model are of the form (1,v),where l is a control vector indicating the current con-  trol node for each component of the network and v is an assignment giving the current value for each clock and inte- ger variable.The initial state of a UPPAAL model consists of the initial node of all components⁵and an assignment giving the value 0 for all clocks and integer variables.A  UPPAAL model determines the following two types oftran- sitions between states:

Delay transitions As long as none of the invariants of the 	control  nodes  in  the  current  state  are  violated,time may progress without affecting the control node vector 	and with all clock values incremented with the elapsed duration  of  time.In  figure  1,from  the   initial  state 《(A0,BO),x=0,y=0,n=0)time      may       elapse      3.5

5indicated graphically by a double circled node.

time  units  leading  to  the   state((A0,BO),x=3.5,y= 3.5,n=0).However,time  cannot  elapse  5  time  units as this would violate the invariant of B0.

Action transitions If two complementary labeled edges of two different components are enabled in a state then they  can   synchronize.Thus   in   state<(A0,BO),x   = 3.5,y   =3.5,n    =0)the   two    components   can   syn- chronize  on  a  leading  to  the  new  state((A1,B1),x= 0,y    =0,n=5)(note    that    x,y,and    n    have    been appropriately updated).If a component has an inter- nal edge enabled,the edge can be taken without any synchronization.Thus      in      state〈(A1,B1),x=0,y=  0,n=5),the  B-component  can  perform  without  syn- chronizing  with  A,leading  to  the  state((A1,B2,x= 0,y=0,n=6).

Finally,in  order  to  enable  modeling  of  atomicity  of transition-sequences  of  a  particular  component(i.e.with-  out time-delay and interleaving of other components)nodes may be marked as committed(indicated by a c-prefix).If one of the components in a  state is in a control node la- beled  as  being  committed,no  delay  is  allowed  to  occur and  any  action  transition(synchronizing  or  not)must  in- volve the particular component (the component is so-to- speak  committed  to  continue).In  the  state  <(A1,B1),x=

0,y=0,n=5)B1   is   committed;thus   without    any    delay the next transition must involve the B-component.Hence the two first transitions of B are guaranteed to be performed atomically.Besides ensuring atomicity,the notion of com- mitted nodes also helps in significantly reducing the space- consumption during verification.

In this section and indeed in the modeling of the au- dio/video protocol presented in the following sections,the values of all clocks are assumed to increase with identical speed  (perfect  clocks).However,UPPAAL  also  supports analysis of timed automata with varying and drifting time- speed of clocks.This feature was crucial in the modeling and analysis of the Philips Audio-Control protocol  [3]us- ing UPPAAL.

##### 3.Informal  protocol  description

In this section we provide an informal presentation of the device control protocol,which is used in existing B&O au- dio/video equipments.The description is split into protocol environment,protocol  syntax,and  dynamic  protocol  rules as advocated in[11].

##### 3.1.Protocol environment

The audio/video components in a B&O system are in- tegrated  through  a  broadcast  network,called  the  bus,for

4

Authorized  licensed  use  limited  to:BEIHANG  UNIVERSITY.Downloaded  on  March  21,2026  at  09:02:51  UTC  from  IEEE  Xplore.Restrictions  apply.

command exchange as indicated in figure 2.Examples of commands are start and stop of a VCR initiated via a re-  mote control⁶.Because the bus is shared,there is a risk of collision between component transmissions,and the proto-  col rules must ensure that collisions are recognized by all  involved componenits in order to prevent data loss or dupli-  cation.

##### 3.2.Protocol   syntax   and   encoding

1(5V) 0(0V)

1562μs              1562*iμs                       1562μs

Figure 3.Physical representation of aT;mes- sage.

The  components  exchange  information  via  so-called frames,where  each  frame  consists  of  a  number   of  T- messages following the abstract syntax:

frame:=T{Ti|T₂|T₃}≥15T₄

So,a frame consists of aTs,followed by a sequence of at  least   15⁷symbols  over  the  set{Ti,T₂,T₃}and  terminated  by  a  T4.The   Ti's  have  the   following  roles:T₅indicates the start of a frame (used for bus reservation);T₄indicates the termination of a frame (used for bus release);and Ti, T₂,and T₃are used for the actual frame data.The detailed  rules for bus reservation and release are given in section 3.3.

# Figure 2.Example B&O configuration

Each T-message(T;)is represented on the bus as voltage levels(0 Volts and 5 Volts)according to the pattern in fig- ure 3.The figure shows that the Ti's are separated by OV for   1562μs-the   so-called  protocol  period.The  Ti's   are identified by the length of the 5V signal between the OV pe- riods.Besides the T;'s,there is an additional pattern called a jamming signal,which is defined as a OV signal for 25ms.

Each component outputs to and reads from the bus via  a one-bit register,wlhere 0 represents OV,and 1 represents 5V.When two or more components are accessing the bus, the OV has priority,that is,the bus changes states according to a logical and as described in figure 4.For the remainder

6Typical   devices    are    TV-sets,VCRs,radios,tape   recorders,CDs,active loudspeakers   etc.

7The    header     size     of    a     frame.           A     header     consists     of

(format,address,command)

of this paper,we use 0 and  1 to denote both the register values and the voltage levels of the bus.

| current bus state | component output, new bus state | component output, new bus state |
| --- | --- | --- |
| 0(0V) 1(5V) | 0,0 0,0 | 1,0 1,1 |

## Figure 4.Rules for changes of bus state

##### 3.3.Protocol     rules

Below we describe  (in  an  informal way)the different rules,which must be obeyed when the bus is accessed by a component.We only deal with the sender aspects of a component,as  the  receiver  part  is  straightforward.Please observe that  each  component has its  own  clock-running independently of all other clocks in the  system.In order to structure the descriptions,we define the following meta phases for a component:The idle phase,where it waits for a new frame to become ready for transmission,the initial- ization phase,where it waits for bus reservation,the trans-  mission phase,where the frame transmission takes place, and the collision handling phase,which is entered after a collision detection.

Bus Reservation Rule A network component reserves the bus by issuing a T5 and releases the bus by issuing a T₄or by detecting a collision and issuing a jamming signal.That  is,if  a   component  has   issued  a  Ts,all other components consider the bus as being reserved.

Frame Gap Rule A network component must ensure the duration  of at  least  50  ms  between  its  transmitted frames.However,if a component has generated ajam- ming signal,it may resend its(destroyed)frame imme- diately after the jamming signal.

Frame Initialization Rule When a frame becomes ready for transmission (in the idle phase),the sending com- ponent  delays  for  781μs(the  reaction  delay),enters

5

the initialization phase,and waits for bus reservation. When  reservation  is  possible(i.e.a  T  has  not  been detected on the bus),the component must wait for ad- ditional 2 periods and check that the bus state is 1 dur- ing the final 781μs of these 2 periods.If this is not the  case,bus reservation  is retried.Otherwise,another 781μs  is  awaited,and  the  transmission  phase  is  en- tered,starting the transmission of aT5.

Bus Sample Rule A sender must sample the bus contents for each period (S₁-points in figure 5)and in the mid- dle of each period (S₂-points in figure 5).

Bus Output Rule A sender must issue output to the bus in the beginning of each sample period(the W-points in  figure  5).For  a  given  period,the  condition  0< (W-S₁)<600μs   must   be   satisfied.8In   the   actual model,we    have    estimated    the    quantity(W-S₁)to 40μs—the so-called output-delay of the protocol.

Collision  Detection  Rule  A  sending  component  must  check the bus for collision at each  S₂-point(see fig- ure  5).For  a  given  period,s₁and  s₂denote  the  bus values  sampled  at  points   S₁and  S₂.Furthermore,pn  and pf denote the values output to the bus from the component at points W of the given period and its pre- decessor.A transmission is collision free,if the condi- tion  ps=S1^Pn=S2  is  satisfied  for  each   S₂-point. If this is not the case,the sender enters the collision handling phase.

Collision Handling Rule Due to the priority between volt- age levels,a collision can only occur,when 0 is sam- pled from the bus.Moreover,if the duration of such an  (inconsistent)0  signal  is  less  than  3  periods,the rule is that the component must issue a jamming sig- nal  and  thereafter  reenter  the  initialization  phase.If the duration is at least 3 periods,another component is jamming.The rule  is that the  sending(non-jamming) component must wait for 18 periods after the 0signal has disappeared from the bus,and thereafter reenter the initialization phase.This delay gives a jamming com- ponent the possibility to retransmit its frame without further collisions.

Transmission Stop Rule Whenever a collision has been de- tected,the component must stop from issuing further bus outputs(and enter the collision handling phase). In this way,it becomes possible to detect if the colli- sion is caused by the jamming of another component.

Detection Stop Rule The final collision detection during frame transmission is the detection performed 781μs after the first 0 signal of the terminating  symbol T₄ .

8Due  to  the  physical  laws  of  how  fast  the  bus  can  change  its  state.

781μs

40μs

S₁w

S₁w             S₁W   S₂

Pf                            51   Pn

s2:bus  sample  at  S₂

s1:bus  sample  at  S₁

Pn:present bus output at W

Pf:previous bus output at W

Figure  5.Relative  ordering  of  the  variables involved in the collision detection performed at the rightmost   S₂-point

Put in another way:When the detection has success- fully passed both the period of the leading 0 of T₄ and also the successor period,the detection must be stopped.This   rule   avoids   'false'collisions,i.e.colli- sions,that are detected after the final 0 of a frame.

Protocol Correctness A protocol implementation is correct with respect to collision if the following two condi- tions  are  satisfied:(1)if  the  frame  transmitted  by  a sender Xis destroyed(by another sender),then sender X  shall  detect  this;and  (2)if  one  sender  detects  a collision,then  all  other  simultaneously  transmitting senders should detect it.

##### 4.A   validated   formal   model   of   the    protocol

From the informal description given in the previous sec- tion it is by no means easy to determine whether the proto- col is correct,i.e.satisfies the Protocol Correctness criteria. Thus,in this section we develop a model of the protocol in the UPPAAL language in order to enable a formal auto- mated verification of its correctness using the UPPAAL tool set.We will refer to this model as validated,meaning that it has been approved by B&O as being a correct abstraction of the existing implementation.

The model is-as all models-an abstraction of the real implemented protocol in the sense that it leaves out details regarded  as  unimportant  for  the  verification  task.In  our case,an additional challenge in choosing abstraction is the need to reduce the state space to search,and hence to reduce time and space consumption during the automatic verifica- tion.

The construction of a model was an iterative process.

Several issues had to be right.First of all,the model should be  valid,reflecting  the  code  in  the  protocol,and  not  do

6

Authorized licensed use limited to:BEIHANG UNIVERSITY.Downloaded on March 21,2026 at 09:02:51 UTC from IEEE Xplore.Restrictions apply.

something  different.Second,the  model  should  be  as  ab- stract as possible tc make verification efficient,but detailed enough in order to catch the error,the nature of which we were not aware.Third,the correctness criteria should itself be valid,reflecting a desired property;and fourth,the cor- rectness criteria should be such that the yet unknown error could be  caught.The  correctness  criteria  went  through  a couple of iterations,and was constantly under debate.

We present the complete validated model of the proto- col,and  from this we  shall then  derive  a reduced model to which the UPPAAL verifier is applied.This reduction is done basically by limiting the number of frames a sender can transmit;and also by limiting the contents of the in- dividual frames:the number of contained T-messages,and their kind.Even with these reductions the protocol will turn out to exhibit erroneous behavior.

##### 4.1.Overview

The protocol is modeled in UPPAAL as a network of 9 timed automata(figure 6),which can be divided into three groups:a bus,a sender system named A,and a sender sys- tem  named  B.Note  that  there  are  no  frame-receivers,as these are not regarded important for the verification task in hand.The sender systems are completely symmetric in their construction,hence,we shall only describe one such, namely system A.

which is activated from the sender at S₂-points,represents the collision detection algorithm.

The frame generator and observer are part of what we will call the environment,hence in principle not compo- nents  of the  implemented  protocol.The  frame  generator basically generates the O's and 1's of a frame to be output by the sender,hence it models the signals coming for exam- ple from a remote control unit.The observer is purely used  to formulate the correctness criteria.

The components communicate via channel synchroniza-  tions  and  via  shared  variables.The  figure  illustrates  the channel connections by arcs going from one component(the one  that  does  a  send"!")to  another(the  one  that  does a receive “?”).As an example,Sender_A reads the current  value of the bus by receiving on either channel zero(value is  0)or  channel  one(value  is   1),whichever  is  enabled.In addition,for each component it is shown(written inside the box)which variables it acccsses in which manner.A vari- able x is in bold(x)if it is assigned to,and in normal font  (x)if it  is  only  read  from.Finally,if  a  variable  is  local, it is in italic (x).Note,that by convention a variable may be mentioned in several components if they  share it.In a  few cases,variables that are only initialized in a component have been omitted for clarity.

##### 4.2.The    bus

Detector_A

Detector.B

B.Pr

A.Pn

B.S1

B.S2

1B-hock Sender.B

A.P   A.Pn A.S1

B.Pa

A.S2

Frame.Gcneralor.B

A.00

A.Pn A.SI  A.S2

mec A.Pn

A.start A.stop  A.eof

B.starnt

A.stor

A.res

A.cn

A.start

9l

B  res

B.Pn

B.stop Beof

B_P  B.Pn B.SI  BS2  B_c

BJm

B_mSg B_T4

B.Pn  B.SI   B_52

AJM5g A.T4

B.diff

A.Pn

B.Pn

## Figjure 6.The Protocol

The   sender   system  A  consists  of  four  automata:a sender Sender A,a detector Detector A,a frame generator Frame_Generator A and an observer Observer.A.The pro- tocol itself(which is the one implemented in assembler),is here modeled by the sender and the detector.The sender is the key component of the system,and is responsible for transmitting  the  frames  over  the  bus,while  the  detector

The status of the bus is decided by two variables,A_Pn and B_Pn,representing the bus registers,as shown in fig- ure  7.The  two  variables  (initialized  to   1)are  set  by  the sender systems at W-points by the sending system perform- ing  one  of  the  assignments  A_Pn   :=0  or  A_Pn   :=1. The senders can sample the actual bus contents by synchro- nizing on channels zero and one respectively.

### Figure 7.The Bus

##### 4.3.The   frame    generator

The frame generator,figure 8,is the component that con- cretely sets the bus by assigning values 0 and 1 to the vari- able APn on request from the sender at its W-points.The generator  is  initialized  by  an  A_frame!action  from  the sender,where after each new assignment to A_Pn is trig- gered by an A.new_Pn!action from the sender,until con- trol returns to the start node.The generator decides what values to assign each time it is triggered by the sender.An A_reset!action  from  the  sender  resets  the  frame  gener- ator in case a collision has been detected.Of course one can argue that assigning to A_Pn is not part of the environ- ment;and we could certainly let the generator just produce 0's and  1's,and let the sender perform the assignments to the bus registers.In fact,such a model existed on our way to the current model,which is however smaller in terms of number of variables used.

Frame_GeneratorA

start

first

APow:Pn?

msg                                                    cset_msg

eset Stop

last

C:continue

## Figure 8.The Generator

Besides A_Pn,three other externally visible variables are assigned   to:A_eof,A_stop   and   A_start.First,the   vari- able  Aeof is  set  to  1  as  soon  as  the  last  T₄message  in a  frame  has  been  transmitted.The  sender  will  then  stop transmitting.Second,according to the Detection Stop Rule, the last collision detection is performed 781μs after the 0 period beginning the last T₄message,and is hereafter dis- connected.This is modeled by letting the generator assign the value  1  to  the  variable  A_stop  at  this  point.Finally,

according to the Bus Reservation Rule,a precondition for Sender B to begin transmission of a new frame is that no T₅message has been output by Sender_A trying to reserve the bus.Hence,an accurate model would here let Sender B sample the bus to detect T₅'s.This complicates the model unnecessarily,and as an abstraction,we let system A set the variable A_start to  1 when system A has transmitted aT₅ (to keep the graph simple:at every output of a 0 ending a

T-message),and clear it again after the last T₄,when the bus is released.Sender B can then read this variable;and vice versa.

Three local variables A_no,Amsg and A_T4 are used to control the flow of the generator.A frame consists of a sequence of T-messages,which we number from  1 and up.The current T-message number is stored in the variable A_no.The variable A.msg  contains the remaining length (in terms of periods)of the current message;that is:the re- maining number of 1's to be output.Recall,that the T₅start message consists of 1's for ten periods (of 1562μs)or sim- ply ten  1's;hence this variable is initialized to  10.Finally, the variable AT4 is set to  1  when the last T₄message is transmitted,just to invoke the exit of the frame generation.

As long as there are messages to transmit,control re- turns to the msg node.From there the upper right loop is entered each time a 0 is output,and at the same time a non- deterministic  choice  is  made  of  a  new  message(length). Note that the lengths of T-messages (in terms of periods, and hence the number of 1's to be output)are as follows: T₁:2,T₂:4,T₃:6,T₄:8,and          T₅:10.The         model          is limited to transmit minimum 17 and maximum 20 messages (including the startingT5 and the endingT₄).This is to limit the search space.The lower right loop is entered for each 1  output  to  the  bus,calculating  the  value  of  the  A.stop variable each time:when there are less than seven  1's left to be output of the last T₄message,collision detection is disconnected.

Note,that the frame generator can be regarded as provid- ing three procedures (the channels),which will be “called” from the  sender.   The  intention  is  that  when the  sender  “calls”one   of  these   procedures,the   sender   waits   until  the  “procedure's  return”.To  model  such  procedure-calls  (which are to be performed atomically)in UPPAAL,we have  used UPPAAL's committed nodes.This is even more the  case for the detector described below.

##### 4.4.The     detector

The detector represents the collision detection algorithm, and is to be regarded as a procedure,which,according to  the Collision Detection Rule,is“called”from the sender at  S₂-points,through      an      A_check!action.As“arguments” it  takes  the  samples  s1  and  s₂represented  by  the  global  variables A_S1 and AS2;and the outputs pf and pn repre-

8

Authorized    licensed     use    limite    to:BEIHANG     UNIVERSITY.Downloaded    on     March    21,2026     at    09:02:51     UTC    from     IEE    Xplore.Restrictions     apply.

sented by the global variables A.Pf and A_Pn,where after it checks the relationship between these values.The result of the check is written into the variables A_err and A_res. Basically A_err counts the number of O's sampled,while A_res is set to 0 if no action is to be taken,1 if sender A should jam,and finally 2 if another sender(B in this case)is jamming.In the latter two cases,sender A should react.The detector is fully made up of committed nodes,hence it con- sumes  no  time,and  "returns"instantly  to  the  wait_call  node after being activated.The graph corresponds closely to a flowchart extracted from the assembler code.The reader is not supposed to grasp the details.

Detector_A

c:ex_SIC

2:3

cex_Pn

A.S2    ==1               cex¹_S¹

A_S2    --0

ceaxL_S2

³2

ccale     e

c.ex_s2( 82:9

## Figure 9.The Detector

##### 4.5.The     sender

The sender is responsible for triggering outputs to the bus,and is the main and most complicated component,see figure  10.It  has  a  single  clock,named A_c,which mainly is used to model the timer interrupts that arrive with inter- vals of 781 μs.The sender-nodes can be divided into three groups:the initialization phase,the transmission phase,and the collision response phase(entered when a collision has been  detected,and  furthermore  a  response  has  been  de- cided).

Initialization  phase.  This  is  the upper part  of the  dia- gram.The     nodes     ex_startand     other_started     model

the first part of the Frame Initialization Rule (related to the Bus Reservation Rule),which specifies that no frame can be transmitted if a T₅message coming from another sender,B in this case,has been detected on the bus.Recall,that this detection   is   modeled(abstracted)with   the   Bstart   vari- able being set to l by Frame_Generator B.The loop at node other.started  represents  the  fact  that  in  case  a  T₅has been detected,then we wait until a T₄message is received, releasing the bus.This waiting is done by once every 3124 μs to check whether the T₄message has been received;here at this abstract level modeled by B_start being equal to 0 again,where after we proceed with the precondition check.

The    nodes     ex_silencel    and     ex_silence2    model the remaining part of the Frame Initialization Rule,where it is specified that the sender must wait further two periods (2*1562μs)after  the  T₅reservation  check;and  in  the  last 781μs of the second period,the bus must be silent(1).This is modeled by waiting 3*781=2343μs,and then check the bus value at the beginning and at the end of the remaining 781 μs interval.Note how the bus is sampled by synchro- nizing  on  either zero?(bus value  is  0)or  one?(bus value is  1).

Transmission   phase.  This is the mid part of the diagram. The transmission starts in node transmit in case the pre- condition checked in the initialization phase is satisfied. The transition to the check_eof node initializes the frame generator(Frame_Generator.A)via    the     A_frame!action. The sender now enters a loop,where each iteration repre- sents a period of 2*781=1562μs.Basically four variables are assigned to during one iteration of this loop:A.Pn in W-points(as  we  have  seen,by  Frame_Generator  A),APf, to hold the previous old value of A_Pn,and finally AS1 and A_S2 to hold the samples in respectively S₁-points and S₂-points,as recorded in the Bus Sample Rule.

In  the  node  check_eof  it  is  examined  whether  an end  of  frame  has  been  reached,in  which  case  the  stop node  is  entered,and  according  to  the  Frame  Gap  Rule, 50  ms  must  then pass before  a  new  frame  is  transmit- ted.Node  check_eof  furthermore  represents  an   S₁-point where A_S1 is sampled if the frame has not been finished. After the  sampling,in  node  newPn,40μs  elapses  accord- ing to the Bus Output Rule before a new value is output,as- signed to A_Pn by Frame_Generator_A,which is triggered with the Anew_Pn!action.Note that  in  case  the  variable A_err differs from 0,it means that a collision has been de- tected,and according to the Transmission stop Rule,trans- mission should stop,here modeled by just outputting  1's to the  bus.In   the  node  sample,A_S2  is  sampled,reaching node  call_observe.Here  the   observer  and  the   collision detection,in  case  not  disconnected,are  activated.In  the ex_jam node the result A_res of the collision detection is examined,and collision response is begun in case it's dif-

9

Authorized    licensed    use    limited    to:BEIHANG    UNIVERSITY.Downloaded    on    March    21,2026    at    09:02:51     UTC    frrom    IEEE    Xplore.Restrictions    apply.

Sender_A

A_e     ;0

=781)

50000 Ae<=5000)

☆eo=- ¹

c:check_eof 二B

(AP=40)

Ae₅"=786

7

是agart 。-1

ccall_check

nilsile781)

Ae243)

2343

(A=781)

A_observe!

c:call_observe

Ae=28116

### Figure 10.The Sender

ferent from 0,i.e.either is  1  or 2,as described in the next paragraph.

Collision  response  phase.This is the lower part of the dia- gram.Recall that the value of A_res decides the response. When  1,SenderAmust jam for 25ms as  stated in the Col- lision  Handling  Rule.When  2,another(Sender  B)must  be jamming,and we must wait for the bus to be silent,where after 18 periods(28116μs)must pass according to the same rule.

5.Error detection using UPPAAL

In this section we shall describe how the error was found in the validated protocol just presented in the previous sec- tion.First,the  correctness  criteria  will  be  formulated,and second,the result of the verification of this criteria,an error trace,will be explained.

##### 5.1.The    correctness    criteria

The correctness criteria is informally stated in the Pro- tocol  Correctness   statement.It   says,that(1)if  the   frame transmitted by a sender X is destroyed (by another sender), then  sender  X  shall  detect  this;and(2)if  one  sender  de- tects a collision,then every other simultaneously transmit-

10

Authorized  licensed  use  limite  to:BEIHANG   UNIVERSITY.Downloaded  on  March  21,2026  at  09:02:51   UTC  trom   IEEE  Xplore.Restrictions  apply.

ting sender should detect it.In order to formulate rule(1), we must formulate what it means for a frame to be de- stroyed.We  define  a  frame  as  destroyed  if sampled  val- ues differ from output values.Hence,we introduce an ob- server automaton observing this for each sender,and figure 11 shows the observer for SenderA.

model,where basically each sender only transmitted a sin- gle frame of T1 messages,surrounded by a Ts and a T₄ . This was considered harmless as the purpose was to locate an existing error rather than to prove some property uni-  versally  true.The  verifier  rejected  the  stated  correctness criteria as being true,and figure 12 illustrates a condensed version of the error trace produced by UPPAAL⁹in terms of a bus-value diagram.UPPAAL required 6,27 minutes of computation and 32M bytes of memory on a Sparc 10.

Figure  12.The  error  trace  visualized

Figure   11.The  Observer

Recall,that this observer is communicated to from the sender  in  terms  of  an  A_observe!   action  at  each  S₂- point.In receiving this signal,the observer sets the variable A.diff to  1  if and  only  if there  is  a  mismatch  between sampled values and output values.That is,if either ALPf≠ A_S1  or  A_Pn≠A_S2.This  is  formulated  slightly  differ- ent in the automaton since UPPAAL does not allow negation in  edge  guards.Note,that  we  cannot  use  Detector_A  to observe the relationship between sampled and output val- ues,since this is one of the components we want to verify. With the observer,we are sure to know when output 1's have been destroyed by O's from another sender.It can easily be shown,that a frame has only been destroyed if at the end of its transmission A_d:iff equals  1.

The correctness criteria can now be formulated as follows:

A[](A_eof        ==1        imply

(A_diff      ==0      and      B_res      ==0))

In order to understand this property,note that A_eof is set to 1 when A's frame has been sent,that A_diff is set to 1 if A's frame has been destroyed,and finally,that B_res is set to 1 if B has detected a collision.The property then says,that  whenever(A[])a  frame  has  been   sent  (ALeof equals 1),the sent frame must be intact (A_diff equals O), and other senders(B in this case)must not have discovered a collision(B.res equals 0).A  symmetric property  is  also verified for sender system B.

5.2.The    error    trace

In order to obtain a fast feed-back(few minutes)during the debugging of the protocol,we worked with a reduced

It appeared to be the Detection Stop Rule that was un- healthy:collision detection seemed to be disconnected too early with the result of messages being lost.The trace de- scribes  a  scenario,where  SenderA  sends  a  frame  of  ex- actly   15T₁messages,while   Sender   B    sends   16T₁mes- sages.Hence,the  two  frames  are  different,although  they are equal up the the last T₁of A.

Sender B starts exactly 40μs after Sender A.Precisely this delay,which fatally equals the delay between a senders S₁-sampling and its bus output,allows the two senders to proceed without any of them discovering their simultaneous bus access.To see this,consider figure 12 which shows how all the O-periods ofthe two frames are positioned relative to each  other:at  point(a)Sender  A  samples  S₁and  is  ready to output a 0,but the output happens in point(b)due to the output  delay(which  is  also  40μs).In  point  (b)Sender  B now also is ready to sample its S₁value.Now,if SenderA outputs its O before B's sampling,then B will sample a 0 while expecting a1,and B will then recognize the collision. However,if Sender A outputs its 0 after B's sampling,then no collision willbe detected by B.

Hence,there is a non-deterministic outcome of each pair of A  and B0-periods:either A willoutput before B  sam- ples,and a collision is detected by B,or A will output after, and no collision will be detected.This mutual ignorance of the collision continues until sender A terminates its last T₁  message,as illustrated by  figure  12,and  explained  in  the following.

The figure in fact illustrates the beginning of the last T₄  message of Sender A,together with the beginning of yet an- other(the 16th)T₁message of Sender B.Up to that point B has sampled before A has output and no collision has been

9The error trace produced by UPPAALcontained 1998 basic transition- steps.

11

Authorized licensed use limited to:BEIHANG UNIVERSITY.Downloaded on March 21,2026 at 09:02:51 UTC from IEE Xplore.Restrictions apply.

detected.Now,however,at  point  (b),sender  A  comes  first and outputs a 0,and this is detected by B in point(d)when the   collision   detection   is   activated(B_err    :=B_err   + 1).In   point    (e),B    then   decides   to   jam(B_res    :=1), which happens in point (f).This is in fact after the last col- lision detection performed by A in point(L).Hence,sender A never observes the collision,while sender B does.Con- sequently,A's message is lost and is not retransmitted.

Put differently,and  simpler,since  a  sender  disconnects its collision detection early in its T₄message,other senders can start jamming after that point without it being detected. The  trace   violates  as  well   A_diff  ==0  as  B_res  ==0 at  the  point  where  ALeof  ==1:sender  A's  frame  is  de- stroyed(without A detecting it),and sender B has detected the collision.

A question is:"how important is it that sender B starts exactly  40  μs  after  A?”.Well,in  the  case  where  both senders send only Timessages,it is important,since if the delay is less than 40,no collision will ever be detected,and in case the delay is above 40,collision will be detected im- mediately  by  both.This  is  true  in  our  model.In  reality, however,clocksin the various audio/video components may have slightly different,and changing,speeds,so in practise senders do not need to start exactly 40μs apart in order to cause the error.

##### 6.Correcting      the      protocol

Thus,as  explained  in  the  previous  section,the  source of the error was identified as the too early disconnection of the collision detection just after the 0 beginning the last T₄message.That  is:the  last  check  is  performed   781  μs after this 0 has been turned back into a  1,at point  (L)in figure 12.This allows another sender to start jamming after this moment without it being detected by the sender having  disconnected.

The reason for disconnecting at that early moment is to prevent a frame from being sent twice,since if a collision is  detected too late,the frame may in fact have come though, since the collision may not be frame destroying,and a re- transmission  will  then  be  a  duplication.As  an  example, think of a frame with an information contents like:“go one channel forward”.However,it has apparently been discon- nected to early,and hence,a solution to the problem is to move the disconnection to a later moment,but not too far since we still want to avoid a frame duplication.

The solution is to move forward and perform the last collision detection 781 μs before the 0 ending the last T₄ message,at point  (N)in  figure  12.Hence,the  collision  de- tection is then only disconnected during the last 0 of the whole  frame.In our model,this correction must be intro- duced in the frame generators,and figure 13 shows the new Frame_Generator A.

Frame_Generator_A

sSfart

first

- 陪w-Pn?

msg

Figure 13.Generator with A_stop:=1   moved

The  modification  consists  of moving  the  assignment A_stop  :=1  to  a  later  point,namely  to  the  edge  going from  node  msg  to  node  last.That  is,when  the  last  0  in the  last  T₄message  is  output.Consequently,the  previous assignment  to  Astop  must  be  removed  resulting  in  the lower right loop edge leaving and returning to node msg. This edge was before broken into a number of edges over committed nodes,see figure 8.The observer is also discon- nected when the collision detection is(is not shown).

With these modifications,the model was verified correct with respect to the same corectness criteria as presented for the previous model.It required 30 minutes of computation and 90M bytes of memory on a Sparc  10.The model ver- ified was down-scaled to a version where each sender only  transmitted one frame,and where sender A only transmitted Ti messages(surrounded by  a  T₅and  aT₄of course),while sender B could transmit the whole range ofT-messages.

##### 7.C  onclusions

The case study clearly showed how model checking can be a help in tracking down undesired behavior in a highly non-deterministic real-time system.The example illustrated the conditions of a real-life problem in the sense that the source of the error (that messages were occasionally lost) was unknown to us,and hence it was not clear at what ab- straction level the model should be formulated.This ques-

12

Authorized    licensed    use    limited    to:BEIHANG    UNIVERSITY.Downloaded    on    March    21,2026    at    09:02:51     UTC    frrom    IEEE    Xplore.Restrictions    apply.

tion of abstraction level was also central in the formulation of the  correctness  criteria.Another  kind  of  abstraction,per- formed  in  a  second  round,consisted   of  reducing  the   ob- tained  model  to  sub-models  that  could  be  verified  within reasonable  time  and   space.It  would  be  useful  to  have  a workbench  which  could  support  easy  derivation  of  verifi- able sub-models from a single full model.It turned out,that  all sub-models were obtained from the full model by adjust- ing  three  different  parameters:(1)whether  or  not  a  sender transmitted  several  frames,or  just   a   single  frame;(2)how many  messages  were  sent  in  a  single  frame;and  finally(3) what messages could!be transmitted in a single frame.

Choosing  the  right  abstractions  were  mainly  an  activ- ity  based  on  intuition,and  the  adjustment  of  the  parame- ters mentioned was in  addition based  on  experiments with the   model   checker.Of   course,with   such   abstractions,one cannot ensure that the protocol in its full complexity is cor- rect  even  if  the  moclel  is  verified  to  be.However,such  a model can be used to reject the protocol in case errors are found,and   this    is   what   happened.Hence,model    check- ing can be seen as a particular advanced kind of debugging where all execution paths in a limited world are examined, rather than some execution paths in a complete world,as in traditional   testing.Furthermore,often   the    abstractions   are of such a harmless kind,that even though the correctness of the model does not imply the correctness of the protocol,it does increase our confidence in its correctness.

Concerning  the  error  trace,it  contained   1998  transition steps,in fact guaranteed to the be shortest trace leading to a state breaking the property to be verified.Examining such a long trace in the simulator turns out to be impracticable,and hence,it was done in an ad hoc fashion(using emacs and its facilities).Research has been initiated to provide means  for trace examination,for example by defining a trace simplifi- cation   language.

Concerning  the  language  for  writing  atomic  edges  be- tween nodes,one could consider a Pascal-like programming language,with   functions,procedures,control    structures   like loop   and   case   constructs,and,of  course,general   datatypes like   enumerated    types,arrays    and   records.The    Murphi- language   [14]-applied   to   a   protocol   verification   in   [9]- could  be  a  good  candidate  for  such  a  language,and  fur- ther research will  explore  this  path.As  a  general  comment on the  graphical  language  for  writing  transition  systems  it was  clearly  concluded,that  this  formalism was  ideal  in  the communication betveen the toolexpert and the protocol de- signer.The simulator additionally turned out to be of a good help when developing and validating the model before ap- plying the verifier.

References

[1]R.Alur,C.Courcoubetis,and   D.Dill.Model-checking    for Real-Time Systems.In Proc.of Logic in Computer Science, pages 414-425.IEEE Computer Society Press,1990.

[2]R.Alur  and  D.Dill.Automata  for  Modelling  Real-Time Systems.In  Proc.of  ICALP'90,volume  443  of  Lecture Notes in Computer Science,1990.

[3]J.Bengtsson,D.Griffioen,K.Kristoffersen,K.G.Larsen,

F.Larsson,P.Pettersson,and W.Yi.Verification of an Au- dio Protocol with Bus Collision Using UPPAAL.In Proc. of CAV'96,volume 1102 of Lecture Notes in Computer Sci- ence.Springer-Verlag,1996.

[4]J.Bengtsson,K.G.Larsen,F.Larsson,P.Pettersson,and

W.Yi.UPPAAL—A Tool Suite for Symbolic and Compo- sitional Verification of Real-Time Systems.In Proc.of the Ist Workshop on Tools and Algorithms for the Construction and Analysis of Systems,volume 1019 of Lecture Notes in Computer Science.Springer-Verlag,May 1995.

[5]J.Bengtsson,K.G.Larsen,F.Larsson,P.Pettersson,and

W.Yi. UPPAALin 1995.In Proc.of the 2nd Workshop on Tools and Algorithms for the Construction and Analysis of Systems,number 1055 in Lecture Notes in Computer Sci- ence,pages 431-434.Springer-Verlag,Mar.1996.

[6]A.Bouali,A.Ressouche,and       V.R.R.de        Simone.The FC2Toolset.Lecture  Notes  in  Computer  Science,1102, 1996.

[7]P.D'Arenio,J.-P.Katoen,T.Ruys,and        J.Tretmans.Mod- elling and Verifying a Bounded Retransmission Protocol.In Proc.of COST 247,International Workshop on Applied For- mal Methods in System Design,1996.

[8]C.Daws,A.Olivero,and  S.Yovine.Verifying  ET-LOTOS programs with KRONOS.In Proc.of7th International Con- ference on Formal Description Techniques,1994.

[9]K.Havelund and N.Shankar.Experiments in Theorem Prov- ing and Model Checking for Protocol Verification.In M.-C. Gaudel and J.Woodcock,editors,FME’96:Industrial Ben- eft and Advances in Formal Methods,volume 1051 of Lec- ture Notes in Computer Science,pages 662-681.Springer- Verlag,1996.

[10]P.-H.Ho and H.Wong-Toi.Automated Analysis of an Audio Control Protocol.In Proc.of CAV'95,volume 939 of Lecture Notes in Computer Science.Springer-Verlag,1995.

[11]G.Holzmann.The Design and Validation of Computer Pro- tocols.Prentice Hall,1991.

[12]H.Jensen,K.Larsen,and  A.Skou.Modelling   and   Analy- sis of a Collision Avoidance Protocol Using SPIN and UP- PAAL.In The Second Workshop on the SPIN Verification System,volume 32 of DIMACS,Series in Discrete Mathe- matics and Theoretical Computer Science.American Math- ematical Society,1996.

[13]M.Lindahl,P.Pettersson,and   W.Yi.Formal   Design   and Analysis of a Gear-Box Controller:an Industrial Case Study using UPPAAL.In preparation.,1997.

[14]R.Melton,D.Dill,C.N.Ip,and  U.Stern.Murphi  Annotated  Reference Manual,Release 3.0.Technical report,Stanford University,Palo Alto,California,USA,July 1996.

[15]R.Milner.Communication and Concurrency.Prentice Hall, Englewood Cliffs,1989.

13

Authorized licensed use limited to:BEIHANG UNIVERSITY.Downloaded on March 21,2026 at 09:02:51 UTC from IEEE Xplore.Restrictions apply.