# Verification of an Audio Protocol  with Bus Collision Using UPPAAL*

Johan   Bengtsson²         W.O.David   Griffioen³,4        Kare     J.Kristoffersen¹

Kim     G.Larsen¹        Fredrik   Larsson²         Paul   Pettersson²         Wang Yi2

IBRICS#,Aalborg     University,Denmark.E-mail:{jelling,kgl}@iesd.auc.dk

2 Department of Computer Systems,Uppsala University,Sweden.

E-mail:{johanb,fredrikl,paupet,yi}@docs.uu.se

³CWI,Amsterdam,The       Netherlands.E-mail:griffioe@cwi.nl

4 Computing Science Institute,University of Nijmegen,The Netherlands.

Abstract.    In  this  paper we  apply the tool UPPAAL¹to an automatic analysis of a version of the Philips Audio Control Protocol with two senders and bus collision handling.This case study is significantly larger than  the  real-time/hybrid  systems  previously  analysed  by  automatic tools.During the case study the tool UPPAAL was extended with a new feature,committed  locations,allowing  efficient  modelling  of broadcast communication.

##### 1      Introduction

During  the  last  few  years  a  number  of tools  for  automatic  verification  of hybrid and   real-time    systems   have    emerged    [DY95,HHWT95,BLL+95,HRP94].These tools  have  by  now  reached  a  state,where  they  are  mature  enough  for  application on  realistic   case-studies;a  claim  we  hope  to   substantiate  in  this  paper.

We  present  an  application  of  our  tool  UPPAAL  to  an  automatic  analysis  of  a version  of  the  Philips  Audio  Control  Protocol  with  two  senders  and  the  con- sequently   caused   problem   of   bus   collision.The   case   study   is   comprehensive compared  with  previous  verification   efforts   of  real-time   and   hybrid   systems, e.g.the   node-space   is    10³times   larger   than    the   case   with    only   one    sender  [BPV94,HWT95,DY95,LPY95].Also,the      number       of      clocks,variables      and  channels  has   increased   considerably.The  bus   collision   version   studied   in   this paper  has  previously  been  verified   in   [Gri94]without  tool   support.

UPPAAL  is  a  tool  for  automatic  verification  of  safety  and  bounded  liveness properties  of  networks  of  timed  automata  and  certain  hybrid  automata.UPPAAL

*This  work  has  been  supported  by  the  European  Communities(under  CONCUR2  and  REACT),NUTEK(Swedish  Board  for  Technical  Development)TFR(Swedish Technical Research Council)and Netherlands Organization for Scientific Research (NWO)under contract SION 612-316-125.

#Basic  Research   in  Computer  Science,Centre  of  the  Danish  National  Research Foundation.

1 The current version of UPPAAL is available on the World Wide Web via the UPPAAL home    page     http://www.docs.uu.se/docs/rtmv/uppaal.

245

contains  a  number  of  features  including  a  graphical  interface  and  automatic generation  of  diagnostic  traces,and  applies  a  combination  of  on-the-fly  state- space examination together with efficient constraint solving techniques [YPD94, BLL+95].

In modelling the Audio Protocol with bus collision it turned out to be conve- nient  in  certain  situations  to  apply  broadcast  communication.An  extension  of  UPPAAL with so-called committed locations allows broadcasts to be modelled as  atomic sequences of two-process  synchronizations,and yields in addition perfor-  mance  improvements.

The verification of Philips Audio Protocol with Bus Collision was carried out using the extended version of UPPAAL installed on a SGI ONYX machine.As  results we have verified the  correctness  of the protocol  for  an  error  tolerance  of 5%on  the  timing,demonstrated  that  correctness  fails  if  the  error  tolerance  is increased to  6%,and  analysed  an  incorrect version  of the protocol  which is  actually  implemented  by  Philips.

2    The  Committed  UPPAAL  model

The basis of the UPPAAL model for real-time systems is networks of timed au- tomata   AD90]with   data    variables   [YPD94].However,to    meet   requirements arising  from various  case  studies,the  UPPAAL model has been  extended  with various new  features  such  as  urgent  transitions  [BLL+95]etc.The  present  case study indicates that we need to  further extend the UPPAAL model with  com- mitted locations to model behaviours such as atomic broadcasting in real-time  systems.Our experiences with UPPAAL show that the notion of committed lo- cations introduced in UPPAAL is not only useful in modelling but also yields  significant improvements in performance.

We assume that a real-time system consists of a fixed number of sequential processes communicating with each other via channels.We further assume that  each communication synchronizes two processes as in CCS.Broadcasting com-  munication can be implemented in such systems by repeatedly sending the same  message to  all  the receivers.To  ensure  atomicity  of such  'broadcast'sequences,  we mark the intermediate locations of the sender as so-called committed loca-  tions which are to be left immediately.

An      Example. To  introduce  the  notion  of  committed  locations  in  timed  au- tomata,consider  the  scenario  shown  in  Figure  1:A  sender  S  is  to  broadcast  a message m to two receivers R₁and R₂.As this requires synchronization between three processes this can not directly be expressed in UPPAAL where synchroniza- tion,as in CCS,is between two processes based on complementarity of actions. However,as  an  initial  attempt  we  may  model  the  broadcast  as  a  sequence  of two two-process  synchronizations,where  first  S  synchronizes  with  R₁on  m₁and then with R₂on m2.However,this is not  an  accurate modelling  as the  intended atomicity  of  the  broadcast  is  not  preserved(i.e.other  processes  may  interfere during  the'broadcast'sequence).To  ensure  atomicity,we  mark  the  intermediate location S2 of the  sender S as a so-called committed location (indicated by the

R2

R21

m1!

m2?

m2!

R22

c:S2

m1?

R12

Fig.1.Broadcasting Communication and Committed Locations.

C:-prefix).The atomicity of the action sequence m₁!m₂!is now achieved by insist- ing that a committed location must be left immediately!This behaviour is quite similar to what has been  called  “urgent  transitions”[HHWT95,DY95,BLL+95] which insists that the next transition taken must be an action(and not a delay). The precise semantics of committed locations will be formalized in the transition rules for networks of timed automata with data variables in the following.

Preliminaries.W     e assume a finite set of clock variables C ranged over by x,y,z  and a finite set of data variables V ranged over by i,j,k.We use G(C,V)to stand  for  the  set  of  formulas  ranged  over  by  g,generated  by  the  following  syntax:  g::=a|g   人g,where a is a constraint of the form:x～n or i～n for a∈C, i∈V,~∈{≤,≥,=}and  n  being   a  natural  number.We   shall   call   elements   of  G(C,V)guards.To  manipulate  clock  and  data  variables,we  use  reset-set  of  the  form:w:=ewhich  is  a  set  of  assignment-operations  in  the  form  w:=e  where  u  is a clock or data variable and e is an expression.A reset-set is a proper reset-set  when the variables are assinged a value at most once,we use R to denote the set  of all proper reset-sets.A reset-operation  on  a  clock variable  should be  in the  form  x  :=n  where  n  is  a  natural  number  and  a  reset-operation  on  an  integer  variable  should  be  in  the  form:i:=c*i+c'where  c,c'are  integer  constants.We  assume that processes synchronize with each other via channels.Let A be a set  of channel names with a subset U of urgent channels on which processes should  synchronize    whenever     possible.We     use     A={α?|a∈A}U{α!|α∈A}u{7}to  denote the set of actions that processes can perform to synchronize with each  other,where r is a distinct symbol representing internal actions.We use name(a)  to  denote  the  channel  name  of  a,defined  by  name(α?)=name(a!)=α .

The   UPPAAL   Model   with    Committed    Locations. An  automaton  A  over actions  A,clock  variables  C  and  data  variables  V  is  atuple(N,Lo,E,No)where N is a finite  set of locations(control-locations)with a  subset Nc  CN being the set  of  committed  locations,lo  is  the  initial  location,and  ECN×G(C,V)×A× R×N  corresponds  to  the  set  of  edges.To  model  urgency,we  require  that  the guard  of  an  edge  with  an  urgent  action  should  always  bet,i.e.if  name(a)∈U and     (l,g,a,r,l'>∈E     then     gt,

In  the  case,(1,g,a,r,l'〉∈E  we  shall  write,l⁹,2,I'which  represents  a  transi- tion from the location l to the location 'with guard g(also called the enabling condition of the edge),action a to be performed and a set of reset-operations r to update the variables.Also,we shall write C(1)whenever l∈Nc.

To model networks of processes,we introduce a CCS-like parallel composition operator for automata.Assume that A1…An are automata.We use Ato denote their parallel composition.The intuitive meaning of A s similar to the CCS parallel composition of A1…An with all actions being restricted,that is,A= (A₁|….An)\A.Thus only synchronization between the components Ai is possible. We shall call A a network of automata.We simply view A as a vector and use Ai to denote its ith component.

Informally,a process modelled by an automaton starts at location lo with all its variables initialized to 0.The values of the clocks increase synchronously with time at location l.At any time,the process can change location by following an edge  provided the current values of the variables satisfy the enabling condition g.With this transition,the variables are updated by r.

A variable assignment is a mapping which maps clock variables C to the non- negative reals and data variablesV to integers.For a variable assignment v and a delay        d,v田d      denotes      the      variable      assignment      such      that(v田d)(z)=v(z)+d                   for

any     clock     variable     z     and(v田d)(i)=v(i)for          any          integer          variablei.This          definition of reflects that all clocks operate with the  same  speed  and that data variables are  time-insensitive.For  a  reset-operation  r(a  set  of  assignment-operations),we use   r(v)to   denote    the   variable    assignment   v'with   v'(w)=val(e,v)whenever w:=e ∈r   and    v'(w′)=v(w')otherwise,where   val(e,v)denotes    the    value   of   e in  v.Given  a  guard  g∈G(C,V)and  a  variable  assignment  v,g(v)is  a  boolean value describing whether g is satisfied by v or not.

A control vector l of a network A is a vector of locations where li is a location of A₂.We shall write  to denote the vector where the ith element li of l is replaced by I.

A state of a network Ais a configuration (I,v)where l is a control vector of A and v is a variable assignment.The initial state of A is<To,vo)where Io is the initial control vector whose elements are the initial locations of  and vo is the initial variable assignment that maps all variables to 0.

To model progress properties,we use the following notion of maximal delay:

So if l is a committed location,there willbe no delay at l.We extend the notion of maximal delay to networks of automata such that synchronization on urgent channels happens immediately:

The semantics of a network of automata A is given in terms of a transition

system with the set of states being the set of configurations and the transition relation defined as follows:

if there  exist

such   that

and  for  all  k  if  C(lk)then  k=i.

if there  exist  and  Tj

such  that

,g;(v),g;(v),r;UT;∈R    and    for    all

Thus,if a  state{I,v)contains  a  committed  location  no  delays  can  take  place. Moreover,any component with committed location must participate in the next (action-)transition.

3     The        Committed        UPPAAL        Implementation

In  the  following,we  present  the  notion   of  committed  locations  in  terms   of the UPPAAL model and its implementation in UPPAAL.In the current version [BLL+95],UPPAAL is able to check for invariance properties,VOβ,and reacha- bility  properties,3◇β,with  respect  to  constraints,β,on  the  admissible  locations of the various components and the values of the clock and data variables.

The  model-checking  is  performed  using  backwards  reachability  analysis  to- gether with  an  efficient  constraint-solving  technique.Also,UPPAAL  adopts  on- the-fly generation of the  state  space in order to  avoid explicit construction of the product automaton and the immediately caused memory problems.

The model-checking is based on a partitioning of the(otherwise infinite)state- space into finitely many symbolic states of the form  [l,U],where U is a simple constraint  system(i.e.a  conjunction  of  atomic  clock  and  data  constraints²).The backwards reachability algorithm checks if a symbolic state  [lg,Uf]is reachable from the initial state[l₀,Uo],where Uo expresses that all clocks and data variables

are initialized to 0.

The  algorithm  essentially  performs  a  backwards,breadth-first  search  of  the symbolic  states.The  search  is  guided  and  pruned  by  two  buffers:Wait,holding the symbolic states waiting to be explored and Passed holding the symbolic states under exploration and already explored.Initially Passed is empty and Wait holds the  single  symbolic  state  [f,U+].The  algorithm  then  repeats  the  following:

1.Pick   a   state   [m,U']from   the   Wait   buffer.

2.Check       ifm=loand        U₀     U'.If   this   is   the   case,return   the   answer   yes.

3.If   m=π    and    U'CU",for    some     [π,U"]in    the     Passed    buffer,drop    [m,U' and  go  to   step   1.Otherwise  save  [m,U']in  the   Passed  buffer.

4.Find all  symbolic  states  [ō,Z]that  lead  to  [m,U']in  one  step  and  store  them in the Wait buffer.

5.If the Wait buffer is not  empty go to step  1,otherwise return the answer no.

2 Simple constraint systems are also know under the term zone.

We  will  not  treat  the  algorithm  in  more  detail  here,but  refer  the  reader  to to  [YPD94,BL96].

Despite its on-the-fly examination of the symbolic state space the above algo- rithm is bound to run into space problems for sufficiently large systems witnessed by an explosion in the size of the Passed buffer,which is used to record the states already visited in  order to  enable pruning  of redundant  examinations(in  3)and  eventually  ensure  termination.The  key  question  is  how  to  limit  the  growth  of this buffer?When using committed locations to ensure atomicity of finite transi- tion sequences of one component(as in modelling broadcast)it obviously suffices to save the symbolic state at the beginning of the sequence.Hence,our proposed solution is simply not to save symbolic states in the Passed buffer which involves committedlocations.We therefore modify step 3 of the algorithm in the following way:

3'.a.If  committed(m)go   directly    to   step   4.

b.Ifm=n   and   U'CU",for   some    [n,U"]in   the   Passed   buffer,drop    [m,U' and go to step  1.

c.If neither  of  the   above  steps  are  applicable,save  [m,U']in  the  Passed buffer.

4     The      Audio       Control      Protocol      with      Bus       Collision

In this section an informal introduction to the audio protocol with bus collision is given.The audio control protocol is a bus protocol,all messages are received by all components on the bus.If a component receives a message not addressed to it,the message is just ignored.Philips allows up to  10  components.

Messages  are  transmitted  using  Manchester   encoding.Time  is  divided  into bit-slots  of  equal  length,a  bit"1"is  transmitted  by  an  up-going  edge  halfway a  bit-slot,a  bit“0”by  a  down-going  edge  halfway  a  bit-slot.If  the  same  bit  is transmitted twice in a row the voltage changes at the end of the first bit-slot. Note that only a single wire is used to connect the components,no extra clock wire is needed.This is one of the properties that makes it a nice (read cheap) protocol.

The protocol has to cope with some problems:(a)The sender and the receiver must agree on the beginning of the  first bit-slot,(b)the length of the message is not known in advance by the receiver,(c)the down-going edges are not detected by  the  receiver.To  resolve  these  problems  the  following  is  required:Messages must  start  with  a  bit“1”and  messages  must  end  with  a  down-going  edge.This ensures that the voltage on the wire is low between messages.Furthermore the senders  must  respect  a  'radio  silence'between  the  end  of  a  message  and  the beginning of the next one.This radio  silence marks the end of a message and the receiver knows that the next up-going edge is the first edge of a new message. It is(almost)possible to decode a Manchester encoded message by only looking to the up-going messages(problem c)only the last zero bit of a message can not be  detected  (consider  messages“10”and“1”).To  resolve  this  it  is  required  that all messages are of odd length.

It  is  possible  that  two  or  more  components  start  transmitting  at  the  same time.The behavior  of the  electric  circuit  is  such  that  the  voltage  on  the  wire will  be  high  as  long  as  one  of  the  senders  pulls  it  high.In  other  words:The wire implements the  or-function.This makes it possible  for  a  sender to notice that someone else is also transmitting.If the wire is high while it is transmitting a  low,a  sender  can  detect  a  bus  collision.This  collision  detection  happens  at certain  points  in  time.Just  before  each  up-going  transition,and  at  one  and three quarters of a bit-slot after a down going edge(if it is still transmitting a low).When a sender detects a collision it will stop transmitting and will try to retransmit  its message  later,

If two messages are transmitted at the same time and one is a prefix of the other,the receiver will not notice the prefix message.To ensure collision detection it is not allowed that a message is a prefix of an other message in transit.In the Philips environment this restriction is met by embedding the source address in each message(and assigning each component a unique source address).

In Figure 2 an example is depicted.Two senders start transmitting at exactly the same time.Because two lines on top of each other is hard to distinguish from  one  line,they  are  shifted  slightly.The  thick  sender  starts  transmitting  “11 …” and the  other“101.…”.At  the  end  of the  first  bit-slot  the  thick  sender  does  a  down,to  prepare  for  the  next  up-going  edge.But  one  quarter  after  this  down  it detects a collision and  stops transmitting.The thin  sender did not notice the  other and continues transmitting.Note that the receiver will decode the message  of the thin sender correctly.

Fig.2.an   example

The  protocol  has  to  cope  with  one  more  thing:timing  uncertainty.Because perfect  clocks  do not  exist  in  the physical world  and because  the protocol  is implemented  on  a  processor that  also  has  to  execute  a  number  of other  time critical  tasks,a  quite  large  timing  uncertainty  is  allowed.A  bit-slot  is  888  mi- croseconds,so the ideal time between two edges is 888 or 444 microseconds.On the generation of edges a timing uncertainty of±5%is allowed.That is:between 844 and 932 for one bit-slot and between 422 and 466 for half a bit-slot.The collision detection just before an up-going edge and the actual generation of this up-going edge must be at most 20 microseconds.The timing uncertainty on the collision detection on one and three quarters after the generation of a down-going edge is ±22 microseconds.Also the receiver has  a timing uncertainty  of ±5%. And,to  complete  the  timing  information,the  distance  between  the  end  of  one

Ahead0

| sendera (Ax,Ad,Anext) SendarB | (Volt,w) UP,DOWN Wire UP,DOWN (Volt,w) |
| --- | --- |

Addl OUT

Check

AINc,expect0,expect1,CAcol1,(as)

(r,1)

Message

(od,Bod,1b,bs)

Receiver

(oaa)

Bhead0

Bhead1

Bempty

Ahead1

Aempty

(Bx,Bd,Bnex*)

VUP,(w)

Add0

Bco¹1

Acoll

Fig.3.Philips Audio-Control Protocol with Bus Collision.

message  and the beginning  of the  next must be  at  least  8000  microseconds(8 milliseconds).

5    A  Formal  Model  of  the  Protocol

To analyze the behavior of the protocol we model the system as a network of six timed automata.The network consists of two parts:a core part and a testing environment.The core part models the components of the protocol to be imple- mented:two  senders,a  wire  and  a  receiver.The  testing   environment,consisting of a  message  generator  and  an  output  checker,is  used  to  model  assumptions about the environment of the protocol and for testing the behavior of the core part.Figure 3 shows a flow-graph of the network where nodes represent timed automata and edges represent synchronization channels or shared variables (en- closed within parenthesis).

The  general  idea  of  the  specification  is  as  follows.The  automaton   Message generates messages for both senders,and also informs the Check automaton on the bits it generated for SenderA.The senders transmit the messages via the wire to  the  receiver.The receiver  communicates  the  bits  it  decoded  to  the  checker. Thus the  Check  automaton  is  able  to  compare  the  bits  generated by  Message and the bits received by Receiver.If this matches the protocol is  correct.

The  senders  A   and  B  are,modulo  renaming(all  A's   in  identifiers  to   B's), exactly  the  same.Because  of  this  symmetry,it  is  enough  to  check  that  the messages transmitted by sender A are received correctly.We will proceed with a short description of each automaton.The definition of these uses  a number of constants that are declared in Figure 4.

The     Senders.Sender A  is  depicted  in Figure  5.It takes input actions Ahead0?, Ahead1?and  Aempty?.The  output  actions  UP!and  DOWN!will  be  the  Manch- ester encoding of the message.The clock Ax is used to measure the time between

UP!and DOWN!actions.The  idea behind the  specification(taken  from  [DY95]) is  that  the  sender  changes  location  each  half  of  a  bit-slot.The  locations  HS (wire  is  high  in  second  half  of bit-slot)and  HF(high  in  first  half  of bit-slot) refer to this idea.Extra locations are needed because of the collision detection.

The clock Ad is used to measure the time elapsed between the detection just be- fore UP!action  and  the  corresponding  UP!action.Furthermore the time  elapsed  since the last DOWN!action is measured.The system is in the locations ar_Qfirst  and ar_Qlast when the next thing to do is the collision test at one or three quar-  ters  of  a  bit-slot.When  Volt  is  greater  than  zero,at  that  moment,the   sender  detects a collision,stops transmitting and returns to the idle location.The clock  w is used to ensure the'radio silence'between messages.This variable is checked  on the transition from idle to ar_first up.

The     Wire. This  small  automaton keeps track  of the voltage on the wire and generates VUP!actions when  appropriate,that  is when  a  UP?action  is received when the voltage is low.

The    Receiver.Receiver  (Figure  6)decodes the bit  sequence using the up-going (modeled as VUP?)changes of the wire.Decoded bits are signaled to the environ- ment  using  output  actions  Add0!,Add1!and  OUT!(OUT!is  used  for  signaling the end of a decoded message).The decoding algorithm of the receiver is a direct translation of the algorithm in the Philips documentation of the protocol.In the automaton each VUP?transition is followed by a transition modeling the decod- ing.This  decoding  happens'at  once'therefore  these  intermediate  locations  are modeled  as  committed  locations.The  automaton  has  two  important  locations, L1 and L0.When the last received bit is a bit“1”the receiver is in location L1, after  receiving  a  bit“0”it  will  be  in  location  L0.The  error  location  is  entered when a VUP?is received much to early.In the complete specification the error location is not reachable,see Section 6.The receiver keeps track of the parity of the received message using the integer variable odd.When the last received bit is  a  bit“1”and  the  message  is  even,a  bit“0”is  added  to  make  the  complete message of odd length.

The     Message     Generator.The  message  generator  generates  messages  of odd length for both sender A and B.Furthermore,the messages generated for sender A, are communicated to the checker.When a collision is detected by sender A this is communicated to the message generator via Acoll?.The message generator will communicate this on his turn to the checker via CAcoll!.Generating messages of odd  length  is  quite  simple.The  only  problem  is  that  it  is  not  allowed  that a message for one sender is a prefix of the message for the other sender.To be more  precise:If only  one  sender  is  transmitting  there  is  no  prefix  restriction. Only when the two senders start transmitting at the same time,it is not allowed that one  sender transmits  a prefix  of the message transmitted by the  other.As mentioned before the reason for this restriction is that the prefix message is not received by the receiver  and  it  is  possible  that  the  senders  do  not  notice  the collision.In other words:The prefix message can be lost.

The    Checker. This  automaton  keeps  track  of  the  bits  in  transit’,that  is  the

bits  that  are  generated  by  the  message  generator  but  not  yet  decoded  by  the  receiver.Whenever a bit is decoded or the end of the message is detected not  conform the generated message the checker enters an error location.Furthermore when sender A detects a collision the checker returns to its initial location.

6     Verification      in      UPPAAL

In this section we verify correctness of the protocol described in previous sections. Recall,that  the  system  is  modelled  as  a  network  of  the  six  timed  automata: Message SenderA,SenderB,Wire,Receiver and Check,and that properties are specified as logical formulas.

The      Correctness      Criteria. The correct behaviour of the protocol is ensured whenever  the  control  of  the  automaton  Check  is  in  location  a  or  start.If  an incorrect  behaviour  is  detected  the  Check-automaton  enters  the  error-location, consequently  property(1)requires  that  the  Check-automaton  is  always  in  loca- tion start or a:

VO(Check.start   V    Check.a)                                              (1)

For the property to be satisfied it is required that the bit sequence received by the  Receiver matches  the bit  sequence  sent  by  SenderA.Furthermore,it  is  also required that the entire bit sequence is received by Receiver (and communicated to the Check-automaton).This is ensured since the error-location of the Check- automaton is reachable if the end of a bit sequence is signalled by Receiver(i.e. OUT!)when unmatched bits  exists  in  the  Check-automaton.

If the Receiver-automaton observes changes of the wire too early in location L1 or L0 control is changed to location error.It is imaginable that error recovery  can be implemented from this location.However,if the other components of the  protocol conform to the specification this location should not be reachable,thus  property(2)requires that the  error-location  in  Receiver is never reachable.

VO┐ Receiver.error                                               (2)

Incorrectness.    Unfortunately  the  protocol  described  in  this  paper  is  not  the protocol  that  Philips  has  implemented.The  original  sender  checked  less  often for  bus  collisions.The  'just  before  the  up  going  edge’collision  detection  was only performed before the first up.(In our modelling this corresponds to modi- fying SenderA and SenderB in the  following way:delete the outgoing transitions of location ar_Qlast_ok and use the outgoing transitions of location ar-up_ok in- stead.)This  version  is  incorrect.In  general  the  problem  is  that  if both  senders are  transmitting  and  one  is  slow  and  the  other  fast,the  distance  can  cumulate to a high value and this can confuse the receiver.UPPAAL generated a counter  example  trace.

Although this problem was known by Philips is it interesting to see how pow- erful the diagnostic traces can be.It enables us not only to find mistakes in the model of a protocol,but also to find design mistakes in real life protocols.

The   Verification   Results.UPPAAL successfully verifies the correctness prop- erties(1)and(2)for  an  error  tolerance  of  5%on  the  timing.Recall  that  SenderA and  SenderB  are,modulo  renaming,exactly  the  same,implying  that  the  verified properties  for  SenderA also  applies  to  the  symmetric  case  for  SenderB.Prop- erty(1)was  verified  in  7.5  hrs  using  527.4  MB  of memory,property(2)in   1.32 hrs using 227.9 MB of memory.

The analysis of the incorrect version of the protocol with less collision detection (discussed above)uses UPPAAL's ability to generate diagnostic traces whenever a  certain  property  is  not   satisfied  by  the   system.The  trace,consisting   of  46 transitions,was generated in 13.0 min using 290.4 MB of memory.Also,attempts to  verify  Property(1)for  the  ull  protocol  with  an  error  tolerance  of  6%on the  timing  failed.The  scenario  is  similar  to  the  one  found  by  Bosscher  et  al. in  [BPV94]for the one  sender protocol.

The properties(above)were verified using the verification  algorithm  for  han- dling committed locations,described in  Section  3,implemented  in  a new proto- type version of UPPAAL,installed on a SGI ONYX.

#### 7    Conclusion

In this paper it is shown to be possible to verify properties of a realistic case study  using  UPPAAL.The  tool  is  able  to  verify  the  correctness  properties  of the Philips Audio Protocol,that is:the receiver only receives messages that are transmitted.Furthermore  the  ability  of  UPPAAL  to  generate  diagnostic  traces proved  very  useful.When  writing  formal   specifications(some)humans  tend  to make mistakes.These mistakes are much easier to locate using a tool that can generate scenarios.This in contrast with using a tool that only provides Yes/No answers to queries.

We proposed the use of committed locations in UPPAAL specifications.Using these  provides  a  significant  efficiency  improvement.Furthermore  the  memory consumption decreases when using committed locations.

Even more important than the efficiency improvement is that committed loca- tions sometimes allow a more natural specification.If a system does a broadcast or multi-way synchronization,this can be modelled much nicer using committed locations.Without committed locations it is not possible in UPPAAL to prohibit other  components  to  perform  actions  during  the  broadcast.With  committed locations these multi communications can be modelled as a single atomic action.

Another option to model broadcast synchronization is to use another synchro- nization mechanism than handshake as used in UPPAAL.We prefer the use of committed locations because it is easier to embed in the model and easier to im- plement.We also think that committed locations and handshake synchronization provide a flexible and expressive model for specifying protocols.

References

[AD90]       R.Alur and D.Dill.Automata for Modelling Real-Time Systems.In Proc.

of ICALP'90,LNCS 443,1990.

[BL96]        Johan  Bengtsson  and  Fredrik  Larsson.UPPAAL  a  Tool for Automatic Ver-

ification   of   Real-time   Systems.Master's   thesis,Uppsala   University,1996.

[BLL+95]   Johan      Bengtsson,Kim     G.Larsen,Fredrik      Larsson,Paul     Pettersson,and Wang  Yi.UPPAAL—a  Tool   Suite   for  Automatic   Verification   of  Real- Time  Systems.In  Proc.of the  Lth  DIMACS  Workshop  on  Verification  and Control  of  Hybrid   Systems,1995.To  appear  in  LNCS,1996.

[BPV94]     D.J.B.Bosscher,I.Polak,and     F.W.Vaandrager.Verification    of     an    Audio-

Control    Protocol.In     Proc.of    FTRTFT'’94,LNCS     863,pages     170-192, 1994.

[DY95]       C.Daws   and   S.Yovine.Two   examples   of  verification   of  multirate   timed

automata  with  KRONOS.In  Proc.of  the   16th   IEEE   Real-Time   Systems Symposium,pages   66-75,December    1995.

[Gri94]        W.O.D.Griffioen.Analysis   of  an   Audio   Control  Protocol  with  Bus  Col-

lision.Master's    thesis,University    of    Amsterdam,Programming     Research Group,1994.

[HHWT95]  Thomas  A.Henzinger,Pei-Hsin   Ho,and   Howard   Wong-Toi.HYTECH:The Next  Generation.In  Proc.of  the   16th   IEEE   Real-Time   Systems   Sympo- sium,pages   56-65,December   1995.

[HRP94]     N.Halbwachs,P.Raymond,and     Y.-E.Proy.Verification     of     linear    hybrid

systems by means of convex approximations.In Static Analysis Symposium, LNCS   864,pages   223-237,1994.

[HWT95]    Pei-Hsin   Ho   and   Howard   Wong-Toi.Automated   Analysis   of  an   Audio

Control   Protocol.In    Proc.of   CAV95,LNCS    939,1995.

[LPY95]     Kim   G.Larsen,Paul   Pettersson,and   Wang   Yi.Diagnostic   Model-Checking

for  Real-Time  Systems.In  Proc.of  the  4th  DIMACS  Workshop  on  Verifi- cation  and  Control  of Hybrid  Systems,1995.To  appear  in  LNCS,1996.

[YPD94]     Wang    Yi,Paul   Pettersson,and    Mats   Daniels.Automatic    Verification    of

| The constants used in the formulas |  |  |
| --- | --- | --- |
| q | 2220 | One quarter of a bitslot: 222 micro sec |
| d | 200 | Detection 'just before'the UP:20micro sec |
| g | 220 | ’Around'25%and 75%of |
| w | 8000 | the bitslot:22 micro sec 0¦The radio silence:8 milli |
| t | 0.05 | sec The timing uncertainty: 5% |

| The constants continued |  |  |
| --- | --- | --- |
| Q2max | 2*q*(1+t) | 4662 |
| Q3min | 3*q*(1-t) | 6327 |
| Q3max | 3*q*(1+t) | 6993 |
| Q5min | 5*q*(1-t) | 10545 |
| Q5max | 5*q*(1+t) | 11655 |
| Q7min | 7*q*(1-t) | 14763 |
| Q7max | 7*q*(1+t) | 16317 |
| Q9min | 9*q*(1-t) | 18981 |
| Q9max | 9*q*(1+t) | 20979 |

Real-Time  Communicating  Systems  By  Constraint-Solving.In  Proc.of  the 7th  International  Conference  on  Formal  Description   Techniques,1994.

| The constants in the automata |  |  |
| --- | --- | --- |
| W | w | 80000 |
| D | d | 200 |
| Almin | q-g | 2000 |
| A1max | q+g | 2440 |
| A2min | 3*q-g | 6440 |
| A2max | 3*q+g | 6880 |
| Q2 | 2*q | 4440 |
| Q2minD | 2*q*(1-t)-d | 4018 |
| Q2min | 2*q*(1-t) | 4218 |

Fig.4.Declaration    of    Constants.

senderA

Aead2m

HF

HS

UP!

Ad<=D

AX:=0

x:=o

AK=2min

效 ：

21B

AX1=0

Acoli!

ar firsLup

Ax=-0

do_down

Ahee:1?

ar_Qfirst_ok

oe-minAcod=A¹mex

Aerpty?

ar_Qlast

8n

a finsLup_ok

Qra

Anext==l AX:0

nhead1? Ad:=0

ar_Qlastok

=A2min

Ad;=0

DOWN!

c:LS

Fig.5.The    SenderA    Automaton.

recelver

VUP?

add:=0

W<=93max Baa:-odd+1

oUT!

oaa20,nAdao?

Add1!

<=85mn                          Adao!

VUP?

ce

Add01

odd;=-odd+1

c:a                p9

Fig.6.The    Receiver    Automaton.