# Formal  Design  and  Analysis  of  a Gear   Controller*

Magnus   Lindahl¹              Paul   Pettersson²              Wang Yi²

¹Mecel         AB,Göteborg,Sweden.E-mail:magnus.lindahlQmecel.se

2 Department of Computer Systems,Uppsala University,Sweden.

E-mail:{paupet,yi}Qdocs.uu.se

Abstract.    In this paper,we report on an application of the validation and verification tool kit UPPAAL in the design and analysis of a proto- type gear controller,carried out in a joint project between industry and academia.We give a detailed description of the formal model of the gear controller and its surrounding environment,and its correctness formal- ized according to the informal requirements delivered by our industrial partner of the project.The second contribution of this paper is a solution to the problem we met in this case study,namely how to use a tool like UPPAAL,which only provides reachability analysis to verify bounded re- sponse time properties.The advantage of our solution is that we need no additional implementation work to extend the existing model-checker, but simple manual syntactical manipulation on the system description.

##### 1      Introduction

Over the past few years,a number of modeling and verification tools for real-time systems  [5,4,3]have  been  developed  based  on  the  theory  of  timed  automata[1]. They  have  been   successfully   applied   in  various   case-studies   [2,6,8].However, the  tools have been mainly used  in  the  academic  community,namely  by  the  tool developers.It  has  been   a  challenge  to   apply  these  tools  to  real-sized   industrial case-studies.In  this  paper  we  report  on  an  application  of  the  verification  tool-kit UPPAAL¹to  a  prototype  gear  controller  developed  in  a  joint  project  between  in- dustry  and  academia.The  project  has  been  carried  out  in  collaboration  between Mecel  AB  and  Uppsala  University.

The  gear  controller  is   a   component  in  the  real-time   embedded   system  that operates  in  a  modern  vehicle.The  gear-requests  from  the  driver  are  delivered over  a  communication  network  to  the  gear  controller.The  controller  implements the  actual  gear  change  by  actuating  the  lower  level  components  of  the  system, such  as  the  clutch,the  engine  and  the  gearbox.Obviously,the  behavior  of  the

*This  work  has  been  supported  by  ASTEC(Advanced  Software  TEChnology), NUTEK(Swedish Board for Technical Development)and TFR(Swedish Technical Research Council).

¹Installation and documentation is available at the UPPAAL home page http://www.docs.uu.se/docs/rtmv/uppaal/.

282

gear controller is critical to the safety of the vehicle.Simulation and testing have been the traditional ways to ensure that the behavior of the controller satisfies certain safety requirements.However these methods are by no means complete in finding errors though they are useful and practical.As a complement,formal techniques have been a promising approach to ensuring the correctness of em- bedded systems.The project is to use formal modeling techniques in the early design  stages  to  describe  design  sketches,and  to  use  symbolic  simulators  and model checkers as debugging and verification tools to ensure that the predicted behavior of the designed controller at each design phase,satisfies certain require- ments under given assumptions on the environment where the gear controller is supposed to operate.The requirements on the controller and assumptions on the environment have been  described by  Mecel AB  in  an  informal  document,and then formalized in the UPPAAL model and a simple linear-time logic based on the UPPAAL logic to deduce the design of the gear controller.

We shall give a detailed description of the formal model of the gear controller  and its surrounding environment in the UPPAAL model and its correctness in the UPPAAL logic according to the informal requirements delivered by Mecel AB.Another contribution of this paper is a lesson we learnt in this case study, namely how to use a tool like UPPAAL,which only provides reachability analysis to  verify  bounded  response  time  properties   e.g.if  f₁(a  request)becomes  true at  a  certain  time  point,f2(a  response)must  be   guaranteed  to  be  true  within a  time  bound.We  present  a  logic  and  a  method  to  characterize  and  model- check response time properties.The advantage of this  approach is that we need no  additional  implementation  work  to  extend  the  existing  model-checker,but simple manual syntactical manipulation on the system description.

The paper is organised as follows:In section 2,we present a simple logic to characterize  safety  and  response  time  properties.Section  3  develops  a  method to model-check  such properties.In  Section  4  and  5  the  gear  controller  system and  its  requirements  are  informally  and  formally  described.In  Section  6  the formal  description  of  the  system  and  its  requirements  are  transformed  using the technique  developed  in  section  2  for  verification  by  reachability  analysis. Section 7 concludes the paper.Finally,we enclose the formal description of the surrounding environment of the gear controller in the appendix.

2    A   Logic   for   Safety  and  Bounded  Response  Time Properties

At the start of the project,we found that it was not so obvious how to formalize (in the UPPAAL logic)the pages of informal requirements delivered by the design engineers.One of the reasons was that our logic is too simple,which can express essentially only invariant properties.After a while,it became obvious that these requirements could be described in a simple logic,which can be model-checked by reachability analysis in combination with a certain syntactical manipulation on the model of the system to be verified.We also noticed that though the logic

is so simple,it characterizes the class of logical properties verified in all previous case studies where UPPAAL is applied(see e.g.[2,6,8]).

2.1     Timed    Transition    Systems    and    Timed    Traces

A timed transition system is a labeled transition system with two types of labels: atomic  actions  and  delay  actions(i.e.positive  reals),representing  discrete  and continuous changes of real-time systems.

Let Act be a finite set of actions and P be a set of atomic propositions.We use R to stand for the set of non-negative real numbers,D for the set of delay actions{e(d)|d∈R},and  ∑for  the  union  ActUD  ranged   over  by   α,α1,α2   etc.

Definition   1.A  timed  transition  system  over  Act  andP  is  a  tupleS=(S,so,→ ,V),where  S  is  a  set  of  states,so   is  the  initial  state,→CS×∑ ×S  is  a  tran-   sition  relation,andV:S→2P   is   a   proposition   assignment   function.              □

A trace o of a timed transition system is an infinite sequence of transitions in     the     form:

where   αi∈∑ .

A position i of o is a natural number.We use o[i]to stand for the ith state of  o,and  o(i)for  the  ith  transition  of  o,i.e.  and

We  use )to  denote  the  duration  of  the   transition,defined  by 	 if  α∈Act  or  d  if  α=e(d).Given  positions  i,k  with  i≤k,we use△(o,i,k)to  stand  for the  accumulated delay of o between the positions i,k, defined by .We  shall only consider non-zeno traces.

Definition    2.A    trace      σ     is    non-zeno      if    for     all     natural     number     T     there     exists     a position  k  such  that  D(o,0,k)>T.For   a  timed  transition   system   S,we  denote by Tr(S)all non-zeno traces of S starting from the initial state  so of S.        □

Note that the timed transition system defined above can also be represented finitely as a network of timed automata For the definition of such networks,we refer to  [7].Let  Abe  a  network  of timed  automata  with  components  A₁…An. We  denote  by  Tr(A)all  non-zeno  traces  of the  timed  transition  system  S  i.e. Tr(A)=Tr(S).

2.2    The    Logic:Syntax     and     Semantics

The logic may be seen as a timed variant of a fragement of the linear temporal logic LTL,which does not allow nested applications of modal operators.It is to express invariant and bounded response time properties.

Definition 3.Assume that GV ranged over by g is a set of clock constraints as defined in  [7]and  P  is  a  finite  set  of propositions  ranged  over  by p,q  etc.Let

284

(l,u)Fg iff g(u) (1,u)Fp iffp∈V(1) (l,u)卡-f iff(l,u)≠f (1,u)Ff₁Af2 iff(1,u)Ff₁and(l,u)f₂ σFInv(f)if Vi:o[i=f ol=fi~≤r f2 iff Vi:(□国)=f₁ →3k≥i:(c[k]=f2 and D(σ,i,k)≤T)

Table 1.Definition of satisfiability.

F₈denote the set of boolean  expressions  over  GVUP ranged over byf,f₁,f2  etc, defined     as     follows:

f                  ::=g|p|-f|f₁^f₂

where  g ∈GV is  a  constraint.and p ∈P  is  an  atomic proposition. We  call  F₈ state-formulas,meaning  that  they  will  be  true  of states.                                   □

As       usual,we       use       f₁Vf₂to        stand       for┐(-f₁A 一f2),and    tt     and    ff    for    -f   Vf    and f  Af  respectively.Further,we   use   f₁→f₂to    denote   -f₁Vf₂ .

Dedeffinineitidoansf4oThelows: set  Ft  ranged   over  by   f,f₁,f₂of  trace-formulas   over   F₈is

φ      ::=Inv(f)|f₁~≤T      f₂

where    T    is    a    natural    number.

Iff₁and f2 are boolean combinations of atomic propositions,we call f₁~≤T f2 a bounded response time formula.                                                                    □

Inv(f)states  that  f  is  an  invariant  property;a   system   satisfies  Inv(f)if  all  its reachable  states  satisfy  f.It  is  useful  to  express   safety  properties,that  is,bad things(e.g.deadlocks)should  never  happen,in  other  words,the  system  should always behave safely.f₁~≤T f2 is similar to the strong Until-operator in LTL, but  with  an  explict  time  bound.In  addition  to  the  time  bound,it  is  also  an invariant  formula.It means that as  soon  as  f₁is true  of a  state,f2 must be true within T time units.However it is not necessary that f₁must be true continously before  f₂becomes true  as required by  the traditional Until-operator.

We shall call formulas of the form f₁~≤T f2 a bounded response time for- mula.Intuitively,f₁may  be  considered  as  a  request  and  f2  as  a  response;thus f₁~≤T f2 specifies the bound for the response time to be T.

We interpret Fs and Ft in terms of states and (infinite and non-zeno)traces of  timed   automata.We  write(l,u)=f  to   denote  that  the  state(l,u)satisfies  the state-formula  f  and  σFφto  denote  that  the  trace  o  satisfies  the  trace-formula φ.The  interpretation  is  defined  on  the  structure  of  f  and  φ,given  in  Table  1. Naturally,if all the traces of a timed automaton satisfy a trace-formula,we say that the  automaton  satisfies  the  formula.

Definition  5.Assume  a  network  of  automata   A  and  a  trace-formula  φ.We write  AFφ  if  and  only  if  oFφ  for  all  o∈Tr(A).                                             □

285

# Fig.1.Illustration of a timed automaton A.

3     Verifying       Bounded       Response       Time      Properties       by Reachability            Analysis

The current version of UPPAAL can only model-check invariant properties by reachability analysis.The question is how to use a tool like UPPAAL to check for bounded response time properties i.e.how to transform the model-checking problem  AFf₁~≤T  f2  to  a  reachability  problem.The  traditional  solution  is to  translate  the  formula  to  a  testing  automaton  t(see  e.g.[6])and  then  check whether the parallel system A||t can reach a designated state of t

We take  a  different  approach.We modify(or rather  decorate)the  automaton A  according  to  the  state-formulas  f₁and  f2,and  the  time  bound  T  and  then construct  a  state-formula  f  such  that

M(A)   Inv(f)iff A   f₁~≤T f₂

where M(A)is the modified version of A.

We  study  an  example.First  assume that  each node  of an  automaton is  as- signed implicitly a proposition at(l)meaning that the current control node is l. Consider an automaton A illustrated in Figure 1 and a formula at(l1)~≤3 at(l2) (i.e.it should always reach l2 from l₁within 3 time units).To check whether A  satisfies the formula,we introduce an extra clock c∈C and a boolean variable ²v₁into  the  automaton  A,that   should  be  initiated  with  ff.Assume  that  the node  l₁has  no  local  loops,i.e.containing  no  edges  leaving  and  entering  l₁.We modify the automaton A as follows:

1.Duplicate  all  edges  entering  node  l₁ .

2.Add-v₁as  a  guard  to  the  original  edges  entering  l₁ .

3.Add  v₁:=tt  and  c:=0  as  reset-operations  to  the  original  edges  entering  l₁ .

4.Add v₁as a guard to the auxiliary copies of the edges entering l₁ .

5.Add  v₁:=ff as  a  reset-operation  to  all  the  edges  entering  l₂ .

2Note that a boolean variable may be represented by an integer variable in UPPAAL.

286

Fig.2.Illustration of a modified timed automaton M(A)of A.

The  modified  (decorated)automaton  M(A)is  illustrated  in  Figure  2.Now we     claim     that

M(A)      Inv(v₁→c≤3)iff        Aat(l1)~≤3        at(l2)

The   invariant   property   v₁→c≤3    states   that    either   -v₁or    if   v₁then   c≤3. There   is   only   one   situation   that   violates   the   invariant:v₁and   c>3.Due   to the  progress  property  of  time(or  non-zenoness),the  value  of  c  should  always increase.It  will  sooner  or  later  pass  3.But  if  l₂is  reached  before  c  reaches  3, V₁will become ff.Therefore,the only way to keep the invariant property true is that  l₂is reached within  3  time  units whenever  l₁is reached.

The above method may be generalized to efficiently model-check response time formulas for networks of automata.Let A(f)denote the set of atomic propo- sitions occuring in a state-formula f.Assume a network A and a response time formulaf₁~≤T f2 For simplicity,we consider the case when only atomic propo- sitions  occur  in  f₁and  f2.Note  that  this  is  not  a  restriction,the  result  can  be easily extended to the generl case.We introduce to A:

1.an  auxiliary  clock  c∈C  and  an  boolean  variable  v₁(to   denote  the  truth value  of f₁)and

2.an  auxiliary  boolean  variable  vp  for  all  p∈A(f₁)UA(f2).

Assume  that  all  the  booleans  of A(f₁),A(f2)and  v₁are  initiated  to  ff.

Let  E(f)denote  the  boolean  expression  by  replacing  all  p∈A(f)with  their corresponding  boolean  variable  vp.As  usual,E(f)[tt/vp]denotes  a  substitution that replaces vp with tt in E(f).This can be extended in the usual way to  set of  substitutions.For  instance,the  truth  value  of  f  at  a  given  state  s  may  be calculated    by    E(f)[tt/vp|p∈V(8)][ff/vp|p∉V(s)].

Now we are ready to construct a decorated version M(A)for the network A.

We modify all the components Ai of A as follows:

1.For  all  edges  of  Ai,entering  a  node  l₁such  that  V(l₁)n  A(f₁)≠0:

-Make two copies of each such edge.

-To  the  original  edge,add  v₁as  a  guard.

-To    the    first    copy,add    -E(f₁)^E(f₁)[tt/vp|p∈V(l₁)]as     a    guard    and c:=0,v₁:=tt   and   vp:=tt    for    all    p∈V(l₁)as    reset-operations.

-To   the    second    copy,add   -v₁^ 一E(f₁)[tt/vp|p∈V(l₁)]as    a    guard    and Vp:=tt  for  all  p∈V(l1)as  reset-operations.

2.For  all  edges  of  Ai  leaving  a  nodel₁such  that  V(l₁)nA(f₁)≠0:add  vp:=ff for   all   p∈V(l₁)as   reset-operations.

3.For   all   edges    of   Ai   entering    a   node   l₂such    that   V(l2)∩A(f₂)≠0:add E(f2)^E(f₂)[tt/v₄lq∈V(l2)]as   a   guard   and   v₁:=ff   as   a   reset-operation.

4.Finally,remove  vp:=tt  and  vp:=ff  whenever  they  occur  at  the   same  edge 3.

Thus,we  have  a  decorated  version  M(A;)for  each  Ai  of  A.We  shall  take M(A₁) ||…|M(An)to be the decorated version of A,i.e.M(A)三 M(A₁)

||…|M(An).

Note that we could have constructed the product automaton of A first.Then the construction of M(A)from the product automaton would be much simpler. But the size of M(A)will be much larger;it will be exponential in the size of the component  automata.Our  construction  here  is  purely  syntactical  based  on  the syntactical  structure  of each component automaton.The  size  of M(A)is in  fact linear in the size of the component automata.It is particularly appropriate for a tool like UPPAAL,that is based on on-the-fly generation of the state-space of a network.For  each  component  automaton  A,the  size  of M(A)can  be  calculated precisely  as  follows:In  addition  to  one  auxiliary  clock  c  and  |P(f₁)U  P(f₂) boolean variables  in M(A),the number of edges  of M(A)is  3×|EA|where  |EAI is the number of edges of A(note that no extra nodes introduced in M(A)).

Note also that in the above construction,we have the restriction that  f₁and f2  contain  no   constraints,but  only  atomic  propositions.The   construction  can be  easily  generalized to  allow  constraints by  considering  each  constraint  as  a proposition  and  decorating  each  location  (that  is,the  incomming  edges)where the  constraint  could  become  true  when  the  location  is  reached.In  fact,this  is what  we  did  above  on  the  boolean  expressions(constraints)E(f₁)and  E(f2). Finally,we have the main theoretical result of this paper.

Theorem     1.M(A)FInv(v₁→c≤T)iff      A=f₁~≤T        f2       for       a       network        of

timed  automata  A   and  a  bounded  response  time  formula  f₁~≤T f2.                  □

4   The Gear Controller

In this section we informally describe the functionality and the requirements of the gear controller proposed by Mecel AB,as well as the abstract behavior of the environment where the controller is supposed to operate.

3 This means that a proposition p is assigned to both the source and the target nodes of the eadge;vp must have been assigned tt on all the edges entering the source node.

Functionality.      The  gear controller changes gears by requesting services pro- vided by  the  components  in  its  environment.The  interaction  with  these  com- ponents is over the vehicles communication network.A description of the gear controller and its interface is as follows.

Interface:    The interface receives service requests and keeps information about the  current  status  of the  gear  controller,which  is  either  changing  gear  or idling.The user of this service is either the driver using the gear stick or a dedicated  component  implementing  a  gear  change  algorithm.The  interface is assumed to respond when the service is completed.

Gear       Controller: The  only  user  of  the  gear  controller  is  its  interface.The controller performs a gear change in five steps beginning when a gear change request is received from the interface.The first step is to accomplish a zero torque  transmission,making  it  possible  to  release  the  currently   set  gear. Secondly the gear is released.The controller then achieves synchronous speed over the transmission and sets the new gear.Once the gear is set the engine torque is increased so that the same wheel torque level as before the gear change is achieved.

Under difficult driving conditions the engine may not be able to accomplish zero torque or synchronous speed over the transmission.It is then possible to change gear using the clutch.By opening the clutch,and consequently the transmission,the  connection between the  engine  and the wheels  is broken. The  gearbox  is  at  this  state  able  to  release  and  set  the  new  gear,as  zero torque and synchronous speed is no longer required.When the clutch closes it safely bridges the speed and torque differences between the engine and the wheels.We refer to these exceptional cases as recoverable errors.

Environment.    The environment of the gear controller consists of the following three  components:

Gearbox:    It  is  an  electrically  controlled  gearbox  with  control  electronics.It provides services to set a gear in  100  to  300 ms and to release a gear in 100 to 200 ms.If a setting or releasing-operation of a gear takes more than 300 ms or 200 ms respectively,the gearbox will indicate this and stop in a specific error  state.

Clutch:    It is  an  electrically controlled clutch that has the  same  sort  of basic services as the gearbox.The clutch can open or close within 100 to 150 ms. If a opening or closing is not accomplish within the time bounds,the clutch will indicate this and reach a specific error state.

Engine:    The engine offers three modes of operation:normal torque,zero torque, and synchronous speed.The normal mode is normal torque where the engine gives the requested engine torque.In zero torque mode the engine will try to find a zero torque difference over the transmission.Similarly,in synchronous speed mode the engine searches zero speed difference between the engine and

the wheels⁴.The maximum time bound searching for zero torque is limited 4 Synchronous speed mode is used only when the clutch is open or no gear is set.

to 400 ms within which  a  safe  state  is  entered.Furthermore,the  maximum time bound for synchronous speed control is limited to 500 ms.If 500 ms elapse the engine enters an error state.

We will refer the error states in the environment as unrecoverable errors since it is impossible for the gear controller alone to recover from these errors.

##### 4.1     Requirements.

In this section we list the informal requirements and desired functionality on the gear controller,provided by Mecel AB.The requirements are to ensure the cor- rectness of the gear controller.A few operations,such as gear changes and error detections,are crucial to the correctness and must be guaranteed within certain time bounds.In addition,there are also requirements on the controller to ensure desired qualities of the vehicle,such as:good comfort,low fuel consumption,and low emission.

1.Performance.     These  requirements  limit  the  maximum  time  to  perform  a gear change when no unrecoverable errors occur.

(a)A gear change should be completed within  1.5  seconds.

(b)A  gear  change,under  normal  operation  conditions,should  be  performed within  1 second.

2.Predictability.      The  predictability  requirements  are  to  ensure  strict  syn- chronization and control between components.

(a)There should not be dead-locks or live-locks in the system.

(b)When the  engine  is  regulating  torque,the  clutch  should  be  closed.

(c)The  gear  has  to  be  set  in  the  gearbox  when  the  engine  is  regulating torque.

3.Functionality.     The  following requirements  are to ensure the desired func- tionality of the gear controller.

(a)It is  able to use  all  gears.

(b)It uses the  engine to  enhance  zero  torque  and  synchronous  speed  over the  transmission.

(c)It uses the gearbox to set and release gears.

(d)It is allowed to use the clutch in difficult conditions.

(e)It does not request zero torque when changing from neutral gear.

(f)The gear controller does not request  synchronous  speed when changing to  neutral  gear.

4.Error      Detection. The gear controller detects and indicates error only when:

(a)the  clutch  is  not  opened  in  time, (b)the  clutch  is not  closed  in time,

(c)the gearbox is not able to set a gear in time,

(d)the gearbox is not able to release a gear in time.

Interface

ReqNeuGear (FromGear)  (ToGear)

GearControl GCTimer

OpenClutch         ClutchIsOpen

CloseClutch           ClutchIsClosed

Clutch

CTimer

(ErrStat)

(ToGear)  		ReqSpeed   	ReqTorque ReqZeroTorque

Engine

ETimer

(UseCase)

GearBox

GBTimer

(ErrStat)

TorqueZero SpeedSet

ReqSet  ReqNeu

GearSet Gearneu

NesGear

Fig.3.A Flow-Graph of the Gearbox System.

5      Formal     Description     of     the      System

To design and analyze the gear controller we model the controller and its envi- ronment in the UPPAAL model [7].The modeling phase has been separated in two  steps.First  a model  of the  environment  is  created,as  its behavior is  spec- ified  in  advance  as  assumptions(see   Section  4).Secondly,the  controller  itself and its interface are designed to be  functionally correct in the given environ- ment.Figure 3 shows a flow-graph of the resulting model where nodes represent automata and edges represent synchronization channels or shared variables (en- closed within parenthesis).The gear controller and its interface are modeled by the  automata  GearControl   (GC)and  Interface(I).The  environment  is  modeled by  the  three  automata:Clutch  (C),Engine(E),and  GearBox(GB).

The system uses six variables.Four are timers that measure1/1000 of seconds  (ms):GCTimer,GBTimer,CTimer  and  ETimer.The  two  other  variables,named FromGear  and  ToGear,are  used  at  gear  change  requests⁵.In  the  following  we describe the five automata of the  system.

Environment.      The  three  automata of the environment model the basic func- tionality and time behavior of the components in the environment.The compo- nents have two channels associated with each service:one for requests and one to respond when service have been performed.

Gearbox:    In automaton GearBox,shown in Figure  8,inputs on channel ReqSet request a gear set and the corresponding response on GearSet is output if the gear  is  successfully  set.Similarly,the  channel  ReqNeu  requests  the  neutral

5The domains of FromGear and ToGear are bounded to {0,….,6},where 1to5 represent gear 1 to gear 5,0 represents gear N,and 6 is the reverse gear.

gear and the response  GearNeu signals if the gear is successfully released.If the gearbox fails to set or release a gear the locations named ErrorSet and ErrorNeu are entered respectively.

Clutch:    The automaton Clutch is shown in Figure 5.Inputs on channels Open- Clutch and CloseClutch instruct the clutch to open and close respectively. The corresponding response channels are ClutchIsOpen and ClutchIsClosed. If the clutch fails to open or close it enters the location ErrorOpen and Er- rorClose respectively.

Engine:    The  automaton  Engine,shown  in  Figure  6,accepts  incoming  requests for synchronous speed,a specified torque level or zero torque on the channels ReqSpeed,ReqTorque  and  ReqZeroTorque  respectively.The  actual  torque level or requested speed is not modeled since it does not affect the design of the  gear  controller⁶.The  engine  responds  on  the  channels  TorqueZero and SpeedSet when the services have been completed.Requests for specific torque  levels(i.e.signal  ReqTorque)are  not  answered,instead  torque  is  as- sumed to increase immediately after the request.If the engine fails to deliver zero torque or synchronous speed in time,it enters location CluthOpen with- out  responding  to  the  request.Similarly,the  location  ErrorSpeed  is  entered if the engine regulates on synchronous speed in too long time.

Functionality.     Given the formal model of the environment,the gear controller has been designed to satisfy both the functionality requirements given in  Sec- tion 4,and the correctness requirements in  Section 4.1

Gear       Controller: The GearControl automaton is shown in Figure 4.Each main loop implements a gear change by interacting with the components of the environment.The designed controller measures response times from the com- ponents  to  detect  errors(as  failures  are  not  signaled).The  reaction  of  the controller  depends  on  how  serious  the  occurred  error  is.It  either  recovers the  system  from  the  error,or  terminates  in   a  pre-specified  location  that points  out  the(unrecoverable)error:COpenError,CCloseError,GNeuError  or GSetError.Recoverable errors are detected in the locations CheckTorque and CheckSyncSpeed.

Interface:    The  automaton  Interface  requests  gears  R,N,1,…,5  from  the  gear controller.A  change  from  gear  1  to  gear  2  is  shown  in  Figure  7.Requests and responses are sent through channel ReqNewGear and channel NewGear respectively.When  a  request  is  sent,the  shared  variables  FromGear  and ToGear are assigned values corresponding to the current and the requested new gear respectively.

⁶Hence,the time bound for finding zero torque(i.e.400 ms)should hold when de- creasing from an arbitrary torque level.

GearControl

ReqNewGear? SyBTimer!=0

c·Initiate

coar!

clutchIsOpen?

CTimer=280

C:ReqNeuGear

eqieur:

GearNeu?

89ed。

ToCear>0

Speedset?

cqS  起 .

GearSet?

eCluchopen2 Begiemer.

GCTimer>200.  GCTimer<=250

&CTimer<=250

GNeuError

9ae6

GCTimer<=200

c.ReqSetGiear2 clutehIaOpen?

8cc200

TeG         0

RegSet!

GcTimer:=0

GCTimer>300

GSeiEror

GearSet?

clutchIsClosed? c:ReqTorqueC

&meLuch!

c:ClutchClose

CTmer<=28。

ReqTorque!

Tocaecut

GCTimer:=0

clutehIsclosed?

e:GeaChanged

acM9852

Fig.4.The Gear Box Controller Automaton.

6    Formal  Validation  and  Verification

In this section we formalize the informal requirements given in Section 4.1 and prove their correctness using the symbolic model-checker of UPPAAL.

To  enable  formalization  (and  verification)of  requirements,we  decorate  the system  description  with  two  integer  variables,ErrStat  and  UseCase.The  vari- able ErrStat is assigned values at unrecoverable errors:1 if Clutch fails to close, 2 if Clutch fails to open,3 if GearBox fails to  set a gear,and 4 if GearBox fails to release a gear.The variable UseCase is assigned values whenever a recover- able error occurs in Engine:1 if it fail to deliver zero torque,and 2 if it is not able to  find  synchronous  speed.The  system  model  is  also decorated to  enable verification of bounded response time properties,as described in  Section 2.

GearControl@lnitiate ~≤1500

((ErrStat=0)→GearControl@GearChanged)                                     (1)

GearControl@lnitiate~≤1000

((ErrStat=0     UseCase=0)→GearControl@GearChanged)    (2)

Clutch@ErrorClose ~≤200 GearControl@CCloseError                                     (3)

Clutch@ErrorOpen ~≤200 GearControl@COpenError                                     (4)

GearBox@Errorldle ~≤350 GearControl@GSetError                                        (5)

GearBox@ErrorNeu ~≤200 GearControl@GNeuError                                      (6)

Inv(GearControl@CCloseError→Clutch@ErrorClose)                                     (7)

Inv(GearControl@COpenError→Clutch@ErrorOpen)                                     (8)

Inv(GearControl@GSetError→GearBox@Errorldle)                                       (9)

Inv(GearControl@GNeuError  →GearBoxQErrorNeu)                                   (10)

Inv(Engine@ErrorSpeed→ErrStat≠0)                                                              (11)

Inv(Engine@Torque   →Clutch@Closed)                                                       (12)

Poss(Gear@Geari)

(13)

Table 2.Requirement Specification

Before  formalizing the requirement  specification  of the  gear  controller  we define  negation  and  conjunction  for  the  bounded  response  time  operator  and the invariant operator defined in Section 2,

A=41^φ2       ifAF₄1and       A₄2 A     -iffA≠φ

We   also   extend   the   (implicit)proposition   at(l)to   at(A,1),meaning   that   the control  location  of  automaton  A  is  currently  l.Finally,we  introduce  Poss(f)to denote   -Inv(一f),f₁☆≤T  f2  to  denote-(f₁~≤T  f2),and  A@l  to  denote  at(A,l). We are now ready to formalize the requirements.

6.1     Requirement         Specification

The  first  performance  requirement   1a,i.e.that   a   gear  change  must  be   com- pleted within  1.5 seconds given that no unrecoverable errors occur,is specified in property  1.It  requires  the  location  GearChanged  in  automaton  GearControl to be reached within  1.5  seconds  after  location Initiate has been entered.Only scenarios without unrecoverable errors are considered as the value of the variable

ErrStat  is  specified  to  be  zero⁷.To  consider  scenarios  with  normal  operation we restrict also the value of variable UseCase to zero (i.e.no recoverable errors occurs).Property  2  requires  gear  changes  to  be  completed  within  one  second given that the  system is operating normally

The properties 3 to 6 require the system to terminate in known error-locations that point out the specific error when errors occur in the clutch or the gear (re- quirements 4ato 4d).Up to 350ms is allowed to elapse between the occurrence of an  error  and  that  the  error  is  indicated  in  the  gear  controller.The  proper- ties  7  to  10  restrict  the  controller  design  to  indicate  an  error  only  when  the corresponding error has arised in the components.Observe that no specific loca- tion in the gear controller is dedicated to indicate the unrecoverable error that may occur when the engines speed-regulation is interrupted (i.e.when location Engine@ErrorSpeed is reached).Property 11 requires that no such location is needed since this error is always a consequence of a preceding unrecoverable error in the clutch or in the gear.

Property  13  holds  if the  system  is  able  to  use  all  gears(requirement  3a).

Furthermore,for full functionality and predictability,the system is required to be dead-lock  and  live-lock  free(requirement  2a).In  this report,dead-lock  and  live- lock properties are not specified due to lack of space.However,property  1(and 2)guarantee progress within bounded time if no unrecoverable error causes the system to terminate.The properties 12 and 14 specify the informal predictability requirements 2b and 2c.

A number of functionality requirements specify how the gear controller should interact with the  environment(e.g.3ato  3f).These  requirements  have  been  used to design the gear controller.They have later been validated using the simulator in UPPAAL and have not been formally specified and verified.

Time     Bound     Derivation.  Property  1  requires  that  a gear change  should be performed within one second.Even though this is an interesting property in itself one may ask for the lowest time bound for which a gear change is guaranteed.We show that the time bound is 900 ms for error-free scenarios by proving that the change  is  guaranteed  at  900  ms(property  15),and  that  the  change  is  possibly not completed at  899 ms(property  16).Similarly,for  scenarios when the  engine  fails to  deliver zero torque we  derive the bound  1055  ms,and  if  synchronous speed is not delivered in the engine the time bound is 1205 ms.

We have shown the shortest time for which a gear change is possible in the three scenarios to be:150 ms,550 ms,and 450 ms.However,gear changes involv- ing neutral gear may be faster as the gear does not have to be released(when changing  from  gear  neutral)or  set(when  changing  to  gear  neutral).Finally  we consider the  same  three  scenarios  but  without  involving  neutral  gear  by  con- straining  the  values  of the  variables  FromGear  and  ToGear.The  derived  time bounds are:400 ms,700 ms and 750.

7Recall that the variable ErrStat is assigned a positive value (i.e.greater than zero) whenever an unrecoverable error occurs.

GearControl@lnitiate ～<900

((ErrStat=0    UseCase=0)→GearControl@GearChanged)(15)

GearControl@lnitiate 4≤899

((ErrStat             =0^UseCase=0)→GearControl@GearChanged)(16)

Table          3.Time          Bounds

Verification  Results.We  have  verified  totally  46  properties  of  the   system⁸ using UPPAAL installed on a 75 MHz Pentium PC equipped with 24 MB of primary memory.The verification of all the properties consumed 2.99 second.

7      Conclusion

In  this  paper,we  have  reported  an  industrial  case   study  in  applying   formal techniques for the design and analysis of control systems for vehicles.The main output  of the  case-study  is  a  formally  described  gear  controller  and  a  set  of formal  requirements.The  designed  controller  has  been  validated  and  verified using the tool UPPAAL to satisfy the safety and functionality requirements on the  controller,provided  by  Mecel  AB.It  may  be  considered  as  one  piece  of evidence that the validation and verification tools of today are mature enough to be applied in industrial projects.

We have given a detailed description of the formal model of the gear con- troller and its surrounding environment,and its correctness formalized in 46 log- ical formulas according to the informal requirements delivered by industry.The verification was performed in a few seconds on a Pentium PC running UPPAAL version 2.12.2.Another contribution of this paper is a solution to a problem we got in this case study,namely how to use a tool like UPPAAL,which only pro- vides reachability analysis to verify bounded response time properties.We have presented a logic and a method to characterize and model-check such properties by reachability analysis in combination with simple syntactical manipulation on the  system  description.

This work concerns only one component,namely gear controller of a control system  for  vehicles.Future  work,naturally  include  modelling  and  verification of the whole control system.The project is still in progress.We hope to report more in the near future on the project.

References

1.R.Alur  and  D.Dill.Automata  for  Modelling  Real-Time  Systems.Theoretical Computer Science,126(2):183-236,April 1994.

8A complete list of the verified properties can be found in the full version of this paper.

2.Johan  Bengtsson,David  Griffioen,Kare  Kristoffersen,Kim  G.Larsen,Fredrik  Lars- son,Paul  Pettersson,and  Wang  Yi.Verification  of  an  Audio  Protocol  with  Bus Collision Using UPPAAL.In Rajeev Alur and Thomas A.Henzinger,editors,Proc. of 8th Int.Conf.on Computer Aided  Verification,number 1102 in Lecture Notes in Computer  Science,pages  244-256.Springer-Verlag,July  1996.

3.Johan  Bengtsson,Kim  G.Larsen,Fredrik  Larsson,Paul  Pettersson,and  Wang  Yi. UPPAAL in 1995.In Proc.of the 2nd Workshop on Tools and Algorithms for the Construction and Analysis of Systems,number 1055 in Lecture Notes in Computer Science,pages  431-434.Springer-Verlag,Mars  1996.

4.C.Daws,A.Olivero,S.Tripakis,and     S.Yovine.The      tool     KRONOS.In     Rajeev Alur,Thomas  A.Henzinger,and  Eduardo  D.Sontag,editors,Proc.of  Workshop  on Verification and Control of Hybrid Systems III,Lecture Notes in Computer Science, pages 208-219.Springer-Verlag,October  1995.

5.Thomas  A.Henzinger,Pei-Hsin  Ho,and  Howard  Wong-Toi.HYTECH:The  Next Generation.In Proc.of the 16th IEEE Real-Time Systems Symposium,pages 56-65, December 1995.

6.H.E.Jensen,K.G.Larsen,and A.Skou.Modelling and Analysis of a Collision Avoid- ance Protocol Using SPIN and UPPAAL.In Proc.of 2nd International Workshop on the SPIN Verification System,pages 1-20,August 1996.

7.Kim  G.Larsen,Paul  Pettersson,and  Wang  Yi.UPPAAL  in  a  Nutshell.To  appear in International Journal on Software Tools for Technology Transfer,1998.

8.Thomas Stauner,Olaf Mller,and Max Fuchs.Using hytech to verify an automotive control   system.In   Proc.Hybrid   and   Real-Time    Systems,Grenoble,March   26- 28,1997.Technische  Universität  München,Lecture  Notes  in  Computer   Science, Springer,1997.

Appendix:The             System             Description

Clutch

m=150)      ETuecnspen!

Open

ErorClose

Eroropen

Closed

e<=150)

eCuch?

150

Fig.5.The   Clutch   Automaton.

Engine

ReqZeroTorque? ETimer:=0

Torque

ETimer>=50 eqTorque?

ToGear>0

ToGear==0

ETimer==2g0

UseCase:

ETimer=900

ToGear==0 ReGSpeed?

Timer:=0

emdse=50

FHSe<=200) ErorSpeed

mdaes)

ETimer==500

ToGear>0 RGqSpeed?

ETimer:=0

r=400

Zero

UseCase:m0

Fig.6.The    Engine    Automaton.

ReqNewGear!

FromGear:=1

NewGear?

ToGear:=2

Gear1                           chkGear12                           Gear2

Fig.7.The   Interface   Automaton:a   gear   change.

GearBox

Neutral                                  GBT1mer>=100

GBTsmer>290

BegNee?.=0

ErrorNeu

GBTimer>=100 GearSet!

=300

BB平Se?:

Errorldle

GearNeu!

# Fig.8.The   Gearbox  Automaton.